VILLA A68

# Material Schedule

PROCUREMENT & EXECUTION · OPTIONS 1 AND 2

Room-by-room approved finishes for the ground floor, upper floor, exterior and outdoor areas. Descriptions reproduce the approved design; undefined technical details are not assumed.

ISSUE  01 · 02 October 2026

STATUS  Design intent schedule

SCOPE  61 room and external-area schedules

## Approval notes

- Reference codes are Villa A68 schedule identifiers, not manufacturer or product codes.

- Descriptions reproduce the approved design specification; no substitute material has been introduced.

- Where no manufacturer is fixed, items state "Supplier/Product Code to be selected" — material type, colour and finish remain as specified.

- All finishes require an approved physical sample and shop drawing before purchase.

- Final manufacturer, batch, technical build-up, fire rating, slip rating and wet-area system remain subject to approval where not stated.

# Schedule reference key

| FAMILY | MATERIAL GROUP | APPROVAL BASIS | STATUS |
| --- | --- | --- | --- |
| ST-01 / ST-02 | Beige marble / dark emperador | Approved natural-stone design intent | Sample required |
| ST-03 / ST-04 | Travertine / limestone | Approved natural-stone design intent | Sample required |
| ST-05 / TL-01 | Sintered stone, quartz / porcelain | Approved engineered-surface intent | Sample required |
| JN-01 / JN-02 | Smoked walnut / oak, teak and wood-effect | Approved joinery design intent | Shop drawing required |
| WF-01 | Paint, plaster, micro-cement and render | Approved colour and finish family | Mock-up required |
| MT-01 / GL-01 | Brass, bronze / clear or fluted glass | Approved metal and glazing intent | Sample required |
| TX-01 / LT-01 | Textiles / decorative and architectural lighting | Approved visual intent | Cut sheet required |
| LS-01 / FL-01 | Landscape planting / safety surfacing | Approved landscape intent | Submittal required |

OPTION 1 · HIGH-END MODERN LUXURY

# Main Entrance & Lobby

Ground floor  ·  4.20 m lobby width · 1.30 m opening to the living wing · stair and guest WC off the same space

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-02 / ST-01 / MT-01 | Dark Emperador Beige marble | Floor / paving | 90×90 cm polished beige marble, straight set, with a central medallion in dark emperador framed by 5 cm brushed brass inlay lines. | 90 cm · 5 cm | Main Entrance & Lobby — Flooring |
| WF-01 / MT-01 | Warm white paint Brushed brass | Walls / paint | Warm off-white polished plaster; one wall in vertical fluted cream stone as the console backdrop. Slim brass reveal at skirting level. | TBC by sample/shop drawing | Main Entrance & Lobby — Walls |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Ceiling | Circular recessed drop with perimeter cove (3000 K indirect), trimless pinhole spots, and a vertical cascading brass-and-glass pendant on the medallion axis. | 3000 K | Main Entrance & Lobby — Ceiling & lighting |
| ST-01 / JN-02 | Beige marble Natural oak / wood | Joinery / woodwork | Full-height flush cloak cupboard, push-to-open, finished in the wall colour so it reads as wall. Floating marble console, 2.20 m. | 2.20 m | Main Entrance & Lobby — Joinery |
| MT-01 / GL-01 / LS-01 | Brushed brass Clear glass | Special finish / equipment | Arched brushed-brass mirror, paired alabaster-glass sconces, one sculptural planter. Nothing else — the lobby stays a threshold, not a room. | TBC by sample/shop drawing | Main Entrance & Lobby — Decorative |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Main Entrance & Lobby — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Main Entrance & Lobby — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Main Entrance & Lobby — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Main Entrance & Lobby — entrance feature wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Main Entrance & Lobby — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Main Entrance & Lobby — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Main Entrance & Lobby — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Main Entrance & Lobby — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Main Entrance & Lobby — all visible metalwork |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Formal Guest Lounge

Ground floor  ·  9.50 m long × 5.65 m deep — south wing, direct off the entrance, as dimensioned on the approved plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-02 / MT-01 / LT-01 | Dark Emperador Brushed brass | Special finish / equipment | 1.60 m wide linear gas fireplace recessed into the panelled feature wall, framed in honed dark emperador with a brushed-brass shadow reveal and a floating stone hearth shelf. Flue routed in the existing partition depth — no structural change. | 1.60 m | Formal Guest Lounge — Fireplace |
| ST-02 / MT-01 / TX-01 | Dark Emperador Brushed brass | Special finish / equipment | Custom modular sofas along three walls, 90 cm deep, deep taupe velvet, tailored no-skirt profile with brushed-brass capped legs. Two dark emperador cube side tables at the corners. | 90 cm | Formal Guest Lounge — Furniture |
| MT-01 / TX-01 | Brushed brass Textiles | Walls / paint | Full-height upholstered fabric panels in warm sand, framed by 20 mm brushed-brass vertical profiles in a tall repeating rhythm — the room's signature treatment. | 20 mm | Formal Guest Lounge — Feature wall |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Ceiling | Stepped perimeter cove with warm indirect light, a large cascading brass-and-glass chandelier centred, and paired wall sconces between the panels. | TBC by sample/shop drawing | Formal Guest Lounge — Ceiling & lighting |
| TX-01 | Textiles | Doors / window treatment | Sheer linen inner layer plus heavy cream blackout drapery, both on a recessed ceiling track — no visible rod. | TBC by sample/shop drawing | Formal Guest Lounge — Curtains |
| ST-01 / TX-01 | Beige marble Textiles | Floor / paving | Polished beige marble with a 4×6 m cream low-pile rug sized so every seat's front legs land on it. | 6 m | Formal Guest Lounge — Flooring |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Formal Guest Lounge — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Formal Guest Lounge — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Formal Guest Lounge — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Formal Guest Lounge — fireplace / TV wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Formal Guest Lounge — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Formal Guest Lounge — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Formal Guest Lounge — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Formal Guest Lounge — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Formal Guest Lounge — all visible metalwork |
| ST-02 / ST-03 / ST-01 / MT-01 | Dark Emperador Travertine | Fireplace | Linear built-in fireplace with bookmatched silver travertine / dark emperador marble slab cladding surround and brushed brass (pvd or solid), satin lacquered trim. Supplier/Product Code to be selected. | 1500–1800 mm firebox | Formal Guest Lounge — fireplace wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Main Family Living Room

Ground floor  ·  9.41 × 6.90 m — north-west, open to the kitchen

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / JN-01 / MT-01 / LT-01 | Beige marble Smoked walnut | Walls / paint | Full-width book-matched beige marble with a recessed screen niche framed in a 20 mm brass reveal; 3.60 m floating smoked-walnut console with a shadow-gap LED beneath. | 20 mm · 3.60 m | Main Family Living Room — TV feature wall |
| ST-01 / LT-01 | Beige marble 2700K LED | Special finish / equipment | A 1.80 m linear gas fireplace sits directly under the screen niche inside the same marble plane, so TV, fire and console read as one architectural composition rather than three added objects. Stone returns 60 cm above the opening; heat break and flue concealed within the wall build-up. | 1.80 m · 60 cm | Main Family Living Room — Fireplace integration |
| JN-02 | Natural oak / wood | Walls / paint | Vertical oak slat panelling (40 mm slats / 20 mm gaps) on dark acoustic backing along the long internal wall — controls echo in a room this size. | 40 mm · 20 mm | Main Family Living Room — Accent wall |
| ST-03 / ST-01 / MT-01 / TX-01 | Travertine Beige marble | Special finish / equipment | Cream bouclé four-seat sofa, four taupe velvet lounge chairs, travertine-and-brass coffee table; second bay with two facing taupe sofas and a marble-top table. | TBC by sample/shop drawing | Main Family Living Room — Furniture |
| ST-01 / MT-01 / LT-01 | Beige marble Brushed brass | Ceiling | Stepped cove with brass reveal, two matching brass-and-alabaster globe chandeliers (one per bay), wall-grazing spots on the marble, two brass floor lamps. | TBC by sample/shop drawing | Main Family Living Room — Ceiling & lighting |
| MT-01 / TX-01 | Brushed brass Textiles | Special finish / equipment | Cream silk-blend rugs per cluster, sheer linen with cream blackout drapery on recessed track, bronze and cream silk cushions. | TBC by sample/shop drawing | Main Family Living Room — Textiles |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Main Family Living Room — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Main Family Living Room — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Main Family Living Room — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Main Family Living Room — fireplace / TV wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Main Family Living Room — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Main Family Living Room — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Main Family Living Room — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Main Family Living Room — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Main Family Living Room — all visible metalwork |
| ST-02 / ST-03 / ST-01 / MT-01 | Dark Emperador Travertine | Fireplace | Linear built-in fireplace with bookmatched silver travertine / dark emperador marble slab cladding surround and brushed brass (pvd or solid), satin lacquered trim. Supplier/Product Code to be selected. | 1500–1800 mm firebox | Main Family Living Room — fireplace wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Dining Area

Ground floor  ·  5.65 m deep × 4.20 m — north-east of the guest lounge, on the kitchen side, exactly as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / JN-01 / MT-01 | Beige marble Smoked walnut | Countertops / stone | Custom 1.80 m round table, solid beige marble top on a sculpted smoked-walnut pedestal; eight cream leather chairs with walnut frames and brass sabots. | 1.80 m | Dining Area — Table & seating |
| ST-01 / JN-01 / GL-01 / LT-01 | Beige marble Smoked walnut | Joinery / woodwork | Full-height smoked-walnut buffet and crockery unit with fluted-glass doors, internal LED, marble top returning 10 cm up the wall. | 10 cm | Dining Area — Joinery |
| MT-01 / LT-01 | Brushed brass 2700K LED | Special finish / equipment | Round multi-tier brass chandelier with alabaster diffusers, hung 80 cm above the table, on a dedicated dimmer. Cove light for ambience only. | 80 cm | Dining Area — Lighting |
| WF-01 / MT-01 / LT-01 | Warm white paint Brushed brass | Walls / paint | Cream plaster with a large antique-bronze mirror panel opposite the window to double daylight and widen the zone visually. | TBC by sample/shop drawing | Dining Area — Walls |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Dining Area — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Dining Area — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Dining Area — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Dining Area — dining feature wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Dining Area — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Dining Area — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Dining Area — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Dining Area — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Dining Area — all visible metalwork |
| ST-01 / JN-01 / JN-02 | Beige marble Smoked walnut | Countertops / stone | Dining table top in beige marble (matching floor family), honed, eased 2 mm edge; walnut/oak sideboard. Supplier/Product Code to be selected. | 20–30 mm | Dining Area — table & sideboard |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Kitchen

Ground floor  ·  North band of the 12.00 m frontage — open to the living and dining areas, sink under the north window, service door to the maid's room

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-01 | Smoked walnut | Joinery / woodwork | Handleless floor-to-ceiling matte cream lacquer for the tall run; smoked-walnut veneer for the island and low units. Appliances fully integrated behind panels except one statement oven column. | TBC by sample/shop drawing | Kitchen — Cabinetry |
| ST-01 / ST-05 | Beige marble Quartz | Floor / paving | Beige marble-effect sintered stone counters (durability over natural marble at the worktop), full-height book-matched splashback behind the hob. | TBC by sample/shop drawing | Kitchen — Surfaces |
| MT-01 | Brushed brass | Countertops / stone | Waterfall edge with a 35 cm overhang for three brass-and-leather stools; recessed brass power module in the top. | 35 cm | Kitchen — Island |
| MT-01 / LT-01 | Brushed brass 2700K LED | Special finish / equipment | Three slim brass pendants over the island, under-cabinet task LED, toe-kick glow for night use. All 3000 K to match the living wing. | 3000 K | Kitchen — Lighting |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Kitchen — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Kitchen — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Kitchen — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Kitchen — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Kitchen — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Kitchen — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Kitchen — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Kitchen — all visible metalwork |
| JN-01 | Smoked walnut | Kitchen cabinets | Smoked walnut veneer tall units + warm taupe matt lacquer base units, handleless J-profile, soft-close hinges & drawers. Supplier/Product Code to be selected. | 18 mm carcass · 600 mm base depth | Kitchen — base, wall, tall units and island |
| ST-01 / ST-05 | Beige marble Quartz | Kitchen countertop | Beige / Calacatta-tone marble or sintered stone, honed. Supplier/Product Code to be selected. | 20 mm + 40 mm mitred edge / waterfall island | Kitchen — worktops and island |
| ST-01 / ST-05 | Beige marble Quartz | Backsplash | Full-height slab in matching beige / calacatta-tone marble or sintered stone. Supplier/Product Code to be selected. | Slab to underside of wall units | Kitchen — cooking and sink walls |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Staircase

Ground floor  ·  2.15 m wide flight · 20 risers · 2.81 m landing

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / MT-01 / LT-01 | Beige marble Brushed brass | Floor / paving | Beige marble treads with a 15 mm brushed-brass nosing inlay; closed risers in matte cream to keep the flight light. | 15 mm | Staircase — Treads & risers |
| JN-01 / GL-01 | Smoked walnut Clear glass | Metal / fittings | Frameless 12 mm clear laminated glass panels in a slim recessed stainless channel, with a smoked-walnut cap handrail on the wall side — contemporary, clean and fully transparent to the lobby. | 12 mm | Staircase — Balustrade |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Special finish / equipment | Continuous LED washing the underside of the handrail plus a vertical cascading brass-and-glass feature in the void, visible from the lobby. | TBC by sample/shop drawing | Staircase — Lighting |
| WF-01 / LT-01 | Warm white paint 2700K LED | Walls / paint | Warm greige limewash plaster with one large sculptural artwork; the understair volume becomes flush concealed storage. | TBC by sample/shop drawing | Staircase — Wall & understair |
| ST-01 | Beige marble | Staircase treads | Solid beige marble treads & risers, honed, 3 anti-slip grooves to nosing. Supplier/Product Code to be selected. | Tread 300 mm · riser 160–170 mm · 20–30 mm | Staircase — all flights and landings |
| GL-01 | Clear glass | Balustrade | Clear frameless laminated toughened glass in recessed floor channel, no handrail rods. Supplier/Product Code to be selected. | 12+12 mm laminated · 1100 mm high | Staircase — open side of flights and landing |
| MT-01 | Brushed brass | Handrail | Brushed brass (PVD or solid), satin lacquered slim wall-mounted handrail. Supplier/Product Code to be selected. | Ø40 mm | Staircase — wall side |
| LT-01 | 2700K LED | Stair lighting | Recessed 2700K LED step lights / under-tread LED strip. Supplier/Product Code to be selected. | — | Staircase — every 2nd step |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish | — | Staircase — stairwell walls |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Staircase — stairwell walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Staircase — double-height stair wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights; feature pendant to double-height void. Supplier/Product Code to be selected. | — | Staircase — soffit and void |
| ST-01 | Beige marble | Skirting | Stringer skirting in matching solid beige marble treads & risers | 100 mm | Staircase — wall stringer |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Guest WCs

Ground floor  ·  Guest WC 2.40 × 1.20 m off the lobby · second WC 1.82 m wide on the maid's-room side, both as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-02 / JN-01 | Dark Emperador Smoked walnut | Special finish / equipment | Full-height book-matched dark emperador on the vanity wall, honed beige stone elsewhere, floating smoked-walnut vanity with an integrated stone basin. | TBC by sample/shop drawing | Guest WCs — Guest powder room |
| ST-03 / JN-02 / MT-01 / LT-01 | Travertine Natural oak / wood | Special finish / equipment | Honed travertine walls and floor, light oak vanity, stone vessel basin, round brass-framed backlit mirror. | TBC by sample/shop drawing | Guest WCs — Second WC |
| LT-01 | 2700K LED | Special finish / equipment | Backlit mirrors, a lit niche for accessories, and perimeter cove — no downlight directly over the mirror. | TBC by sample/shop drawing | Guest WCs — Lighting |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal / fittings | Brushed-brass wall-mounted tapware, concealed cistern with a brass flush plate, wall-hung pan. | TBC by sample/shop drawing | Guest WCs — Fittings |
| ST-01 | Beige marble | Bathroom floor | Beige marble, honed anti-slip (R10) finish. Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Guest WCs — full floor |
| ST-01 | Beige marble | Bathroom walls | Beige marble full-height cladding, honed. Supplier/Product Code to be selected. | 1200×600 mm · 15–20 mm | Guest WCs — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Guest WCs — non-tiled wall areas |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding behind vanity with backlit mirror. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Guest WCs — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Guest WCs — full ceiling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Vanity | Wall-hung vanity, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Guest WCs — vanity wall |
| ST-01 | Beige marble | Countertop | Beige marble top with integrated moulded basin, honed. Supplier/Product Code to be selected. | 20 mm | Guest WCs — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brushed brass (pvd or solid), satin lacquered flush plate & mixers. Supplier/Product Code to be selected. | — | Guest WCs — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brushed brass (PVD or solid), satin lacquered — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Guest WCs — all fittings |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Guest WCs — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Corridors & Transitional Areas

Ground floor  ·  Lobby-to-wing links and the service passage

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / MT-01 | Beige marble Brushed brass | Floor / paving | Continuous beige marble with a 5 cm brass inlay strip marking the transition between the guest wing and the family wing. | 5 cm | Corridors & Transitional Areas — Flooring |
| JN-01 / WF-01 / MT-01 | Smoked walnut Warm white paint | Doors / window treatment | Full-height flush smoked-walnut doors with vertical brass handles; service doors painted flush to the wall so they disappear. | TBC by sample/shop drawing | Corridors & Transitional Areas — Doors |
| LT-01 | 2700K LED | Special finish / equipment | Linear ceiling cove plus discreet pinhole spots grazing the wall — deliberately dimmer than the rooms it serves. | TBC by sample/shop drawing | Corridors & Transitional Areas — Lighting |
| SP-01 | Dark bronze / special | Special finish / equipment | One console with a single sculptural object and one framed abstract per run. Restraint is the point. | TBC by sample/shop drawing | Corridors & Transitional Areas — Art |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Corridors & Transitional Areas — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Corridors & Transitional Areas — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Corridors & Transitional Areas — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Corridors & Transitional Areas — end wall of corridor |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Corridors & Transitional Areas — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Corridors & Transitional Areas — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Corridors & Transitional Areas — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Corridors & Transitional Areas — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Corridors & Transitional Areas — all visible metalwork |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Maid's Room, Service WC & Boiler

Ground floor  ·  Maid's room on the north-east corner as drawn · service WC 1.82 m · boiler 1.60 m recess off the stair core

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-02 | Natural oak / wood | Special finish / equipment | Single bed with an upholstered head panel, full-height oak-veneer wardrobe with an open shelf bay, 1.10 m desk, blackout roller blind on the drawn window. | 1.10 m | Maid's Room, Service WC & Boiler — Maid's room |
| ST-03 / TL-01 / LT-01 | Travertine Marble-look porcelain | Special finish / equipment | Travertine-look porcelain to full height, wall-hung pan with a concealed cistern, compact stone-top vanity — plumbing kept exactly on the drawn wet wall. | TBC by sample/shop drawing | Maid's Room, Service WC & Boiler — Service WC |
| SP-01 | Dark bronze / special | Special finish / equipment | Stacked washer-dryer in a ventilated niche behind flush doors, with the boiler recess off the stair core retained in its drawn position and given a louvred access panel. | TBC by sample/shop drawing | Maid's Room, Service WC & Boiler — Laundry & boiler |
| TL-01 / WF-01 / LT-01 | Marble-look porcelain Warm white paint | Walls / paint | Warm greige paint, matte beige porcelain floor, one cove plus recessed spots at 3000 K to match the rest of the ground floor. | 3000 K | Maid's Room, Service WC & Boiler — Finishes & lighting |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Maid's Room, Service WC & Boiler — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Maid's Room, Service WC & Boiler — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Maid's Room, Service WC & Boiler — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Maid's Room, Service WC & Boiler — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Maid's Room, Service WC & Boiler — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Maid's Room, Service WC & Boiler — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler — all visible metalwork |
| ST-01 | Beige marble | Bathroom floor | Beige marble, honed anti-slip (R10) finish. Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Maid's Room, Service WC & Boiler (service WC) — full floor |
| ST-01 | Beige marble | Bathroom walls | Beige marble full-height cladding, honed. Supplier/Product Code to be selected. | 1200×600 mm · 15–20 mm | Maid's Room, Service WC & Boiler (service WC) — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler (service WC) — non-tiled wall areas |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding behind vanity with backlit mirror. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Maid's Room, Service WC & Boiler (service WC) — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Maid's Room, Service WC & Boiler (service WC) — full ceiling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Vanity | Wall-hung vanity, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Maid's Room, Service WC & Boiler (service WC) — vanity wall |
| ST-01 / GL-01 / LT-01 | Beige marble Clear glass | Shower area | Beige marble walls, fluted marble niche, 10 mm low-iron frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Maid's Room, Service WC & Boiler (service WC) — walk-in shower |
| ST-01 | Beige marble | Countertop | Beige marble top with integrated moulded basin, honed. Supplier/Product Code to be selected. | 20 mm | Maid's Room, Service WC & Boiler (service WC) — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brushed brass (pvd or solid), satin lacquered flush plate & mixers. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler (service WC) — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brushed brass (PVD or solid), satin lacquered — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler (service WC) — all fittings |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Maid's Room, Service WC & Boiler (service WC) — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Central Upper Lobby

Upper floor  ·  3.75 m × 2.50 m clear circulation · four bedroom doors and the stair void

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / MT-01 / TX-01 | Beige marble Brushed brass | Floor / paving | The same 90×90 cm polished beige marble as the ground floor, straight set, with a 3 cm brushed brass inlay border tracing the lobby perimeter. A hand-loomed greige silk-wool runner softens the centre. | 90 cm · 3 cm | Central Upper Lobby — Flooring |
| JN-01 / WF-01 / MT-01 | Smoked walnut Warm white paint | Walls / paint | Warm cream polished plaster with slim vertical panel reveals. All four bedroom doors are full-height smoked walnut, flush with the panelling, with recessed brushed brass pulls — identical detailing so the lobby reads calm. | TBC by sample/shop drawing | Central Upper Lobby — Walls & doors |
| GL-01 / LT-01 | Clear glass 2700K LED | Ceiling | Stepped gypsum perimeter with continuous 3000 K cove, trimless pinhole spots on the doors, one small sculptural glass pendant on axis, plus a dedicated 3000 K under-cabinet LED strip and a backlit open niche in the pantry. Night circuit at 5% on the cove and niche only. | 3000 K · 3000 K · 5% | Central Upper Lobby — Ceiling & lighting |
| ST-05 / JN-01 / MT-01 / LT-01 | Quartz Smoked walnut | Joinery / woodwork | A 1.60 m full-height built-in dry pantry, flush with the wall panelling: handleless beige-taupe lacquer and smoked-walnut lower cabinets with concealed pull-out storage, a 20 mm honed beige sintered-stone worktop with matching full-height splashback, a small undermount sink with a slim brushed-brass mixer, walnut upper cabinets on push-to-open, and a backlit open niche between them for the coffee machine, kettle and crockery. Brushed-brass sockets and USB points are recessed in the niche splashback, with a shadow-gap toe kick and brass reveals framing the unit. | 1.60 m · 20 mm | Central Upper Lobby — Preparation pantry |
| ST-01 / JN-01 / WF-01 / MT-01 / TX-01 | Beige marble Smoked walnut | Special finish / equipment | Beige marble · smoked walnut · cream plaster · brushed brass · greige silk-wool. | TBC by sample/shop drawing | Central Upper Lobby — Materials |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Central Upper Lobby — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Central Upper Lobby — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Central Upper Lobby — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Central Upper Lobby — pantry wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Central Upper Lobby — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Central Upper Lobby — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Central Upper Lobby — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Central Upper Lobby — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Central Upper Lobby — all visible metalwork |
| JN-01 | Smoked walnut | Kitchen cabinets | Built-in pantry, warm beige/taupe matt lacquer lower & upper units with smoked walnut veneer on mr-mdf, matt lacquer 10% sheen open niche. Supplier/Product Code to be selected. | 600 / 350 mm deep | Central Upper Lobby — pantry wall |
| ST-01 | Beige marble | Kitchen countertop | Beige marble (matching floor family), honed, eased 2 mm edge with small undermount sink. Supplier/Product Code to be selected. | 20 mm | Central Upper Lobby — pantry |
| LT-01 | 2700K LED | Backsplash | Matching stone slab with under-cabinet LED. Supplier/Product Code to be selected. | Full height between units | Central Upper Lobby — pantry |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Staircase Landing & Feature Wall

Upper floor  ·  Stair 2.15 m wide arriving on the east side of the lobby

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-04 / LT-01 | Limestone 2700K LED | Walls / paint | Full-height vertically fluted cream limestone, 60 mm pitch, grazed from a concealed slot at ceiling level so the flutes read as light and shadow rather than as texture alone. | 60 mm | Staircase Landing & Feature Wall — Feature wall |
| GL-01 | Clear glass | Metal / fittings | Frameless 12 mm clear laminated glass in a recessed stainless channel with a slim capping rail, continuing the ground-floor stair detail without change. No vertical rods. | 12 mm | Staircase Landing & Feature Wall — Balustrade |
| ST-01 / MT-01 / LT-01 | Beige marble Brushed brass | Floor / paving | 20 mm beige marble treads with a 10 mm brushed brass nosing insert; concealed LED under each nosing on a separate low-level night circuit. | 20 mm · 10 mm | Staircase Landing & Feature Wall — Treads & landing |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Special finish / equipment | One cascading brass-and-glass rod chandelier in the void, set so the lowest element is visible from the ground-floor lobby and the mass sits at eye level on the landing. | TBC by sample/shop drawing | Staircase Landing & Feature Wall — Decorative lighting |
| JN-01 | Smoked walnut | Special finish / equipment | A recessed walnut niche with a lit sculpture, and a slim stone-topped console on the return wall. | TBC by sample/shop drawing | Staircase Landing & Feature Wall — Accents |
| ST-01 | Beige marble | Staircase treads | Solid beige marble treads & risers, honed, 3 anti-slip grooves to nosing. Supplier/Product Code to be selected. | Tread 300 mm · riser 160–170 mm · 20–30 mm | Staircase Landing & Feature Wall — all flights and landings |
| GL-01 | Clear glass | Balustrade | Clear frameless laminated toughened glass in recessed floor channel, no handrail rods. Supplier/Product Code to be selected. | 12+12 mm laminated · 1100 mm high | Staircase Landing & Feature Wall — open side of flights and landing |
| MT-01 | Brushed brass | Handrail | Brushed brass (PVD or solid), satin lacquered slim wall-mounted handrail. Supplier/Product Code to be selected. | Ø40 mm | Staircase Landing & Feature Wall — wall side |
| LT-01 | 2700K LED | Stair lighting | Recessed 2700K LED step lights / under-tread LED strip. Supplier/Product Code to be selected. | — | Staircase Landing & Feature Wall — every 2nd step |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish | — | Staircase Landing & Feature Wall — stairwell walls |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Staircase Landing & Feature Wall — stairwell walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Staircase Landing & Feature Wall — double-height stair wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights; feature pendant to double-height void. Supplier/Product Code to be selected. | — | Staircase Landing & Feature Wall — soffit and void |
| ST-01 | Beige marble | Skirting | Stringer skirting in matching solid beige marble treads & risers | 100 mm | Staircase Landing & Feature Wall — wall stringer |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master Bedroom One — South-West Suite

Upper floor  ·  4.05 m × 5.40 m (4.10 m at the bed wall) · private ensuite 1.80 m × 2.90 m · PC window to the west

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Special finish / equipment | 180×200 king centred on the long internal wall, 1.05 m clear to the wardrobe run, a 1.60 m upholstered bench at the foot. Two floating walnut bedside tables, 55 cm, with concealed sockets. | 200 k · 1.05 m · 1.60 m · 55 cm | Master Bedroom One — South-West Suite — Bed & layout |
| MT-01 / TX-01 / LT-01 | Brushed brass Textiles | Walls / paint | Full-height headboard wall in cream bouclé panels with 15 mm brushed brass vertical reveals, flanked by two backlit alabaster-effect stone panels that act as the room's night light. | 15 mm | Master Bedroom One — South-West Suite — Headboard / feature wall |
| JN-01 / MT-01 / LT-01 | Smoked walnut Brushed brass | Joinery / woodwork | 3.40 m of full-height smoked walnut joinery: two fluted 4-door hinged banks with brass shadow-gap pulls, and a central open dressing niche with a drawer chest, mirror and warm LED wash. | 3.40 m | Master Bedroom One — South-West Suite — Wardrobes & storage |
| ST-01 / JN-01 / MT-01 | Beige marble Smoked walnut | Walls / paint | 65" screen recessed into a warm beige marble slab framed in brass, over a fluted walnut media console with a stone top. | TBC by sample/shop drawing | Master Bedroom One — South-West Suite — TV wall |
| ST-01 / JN-01 / MT-01 / TX-01 | Beige marble Smoked walnut | Joinery / woodwork | 1.40 m bespoke walnut dressing table with a cream marble top, backlit round brass mirror and a bouclé swivel stool at the west window. | 1.40 m | Master Bedroom One — South-West Suite — Vanity |
| LT-01 | 2700K LED | Ceiling | Stepped gypsum with a continuous 3000 K cove, trimless spots washing the joinery, a pair of alabaster pendants at the bedheads, and two-way dimming from the door and both bedsides. | 3000 K | Master Bedroom One — South-West Suite — Ceiling & lighting |
| JN-02 / TX-01 | Natural oak / wood Textiles | Floor / paving | Warm oak herringbone, 90×450 mm, brushed and matt-lacquered. A 2.40×3.00 m hand-tufted greige wool rug under the lower two-thirds of the bed. | 450 mm · 3.00 m | Master Bedroom One — South-West Suite — Flooring & rug |
| TX-01 | Textiles | Doors / window treatment | Recessed twin track: ivory linen sheer plus a taupe blackout drape in a heavyweight matt weave, wall-to-wall, ceiling-hung. | TBC by sample/shop drawing | Master Bedroom One — South-West Suite — Curtains |
| ST-01 / JN-01 / JN-02 / MT-01 / TX-01 | Beige marble Smoked walnut | Special finish / equipment | Cream bouclé · smoked walnut · beige marble · brushed brass · oak · greige wool. | TBC by sample/shop drawing | Master Bedroom One — South-West Suite — Materials |
| JN-01 / JN-02 / LT-01 | Smoked walnut Natural oak / wood | Flooring | Engineered smoked walnut / oak plank floor, matt oiled. Supplier/Product Code to be selected. | 190×1900 mm · 15 mm | Master Bedroom One — South-West Suite — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Bedroom One — South-West Suite — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Bedroom One — South-West Suite — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Master Bedroom One — South-West Suite — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Bedroom One — South-West Suite — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Bedroom One — South-West Suite — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Master Bedroom One — South-West Suite — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Bedroom One — South-West Suite — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Bedroom One — South-West Suite — all visible metalwork |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Wardrobes | Built-in full-height wardrobes, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Bedroom One — South-West Suite — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master One — Dressing Room

Upper floor  ·  2.60 m × 1.95 m · between the bedroom and the ensuite, on the drawn opening

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-01 | Smoked walnut | Joinery / woodwork | Full-height smoked walnut units on both long walls: hanging at 1.05 m and 1.90 m, adjustable shelving, and four soft-close drawer banks with leather-lined jewellery trays. | 1.05 m · 1.90 m | Master One — Dressing Room — Wardrobes |
| MT-01 / GL-01 | Brushed brass Clear glass | Special finish / equipment | Two central bays in fluted reeded glass with brushed brass frames for bags and shoes, each shelf lit from within. | TBC by sample/shop drawing | Master One — Dressing Room — Display sections |
| LT-01 | 2700K LED | Special finish / equipment | 3000 K LED profile behind every hanging rail and under every shelf, on PIR sensors so the units light as the doors open. | 3000 K | Master One — Dressing Room — Internal lighting |
| ST-01 / JN-01 / MT-01 / TX-01 | Beige marble Smoked walnut | Joinery / woodwork | 1.20 m walnut dressing vanity with a cream marble top, backlit brass mirror and an upholstered bouclé stool at the end of the run. | 1.20 m | Master One — Dressing Room — Vanity |
| ST-01 / WF-01 / MT-01 / TX-01 / LT-01 | Beige marble Warm white paint | Walls / paint | Beige marble floor with an inset greige wool rug, micro-cement ceiling with a slim perimeter cove, brass shadow-gap pulls throughout. | TBC by sample/shop drawing | Master One — Dressing Room — Finishes |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Master One — Dressing Room — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master One — Dressing Room — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master One — Dressing Room — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master One — Dressing Room — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Master One — Dressing Room — all wall perimeters |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master One — Dressing Room — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master One — Dressing Room — all visible metalwork |
| JN-01 / MT-01 / GL-01 / LT-01 | Smoked walnut Brushed brass | Wardrobes | Full-height wardrobes, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen fronts; bronze-tint glass display doors, integrated LED, leather-look internal drawers. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master One — Dressing Room — all wardrobe walls |
| ST-01 | Beige marble | Joinery | Dressing island / vanity with beige marble (matching floor family), honed, eased 2 mm edge top and full-length mirror. Supplier/Product Code to be selected. | 20 mm top | Master One — Dressing Room — centre / vanity wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master Ensuite One

Upper floor  ·  1.80 m × 2.90 m · WC, vanity and walk-in shower, on the drawn wet wall

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 | Beige marble | Walls / paint | Book-matched warm beige marble to full height on all wet walls and floor, 20 mm slab vanity top with an integrated basin. | 20 mm | Master Ensuite One — Stone |
| JN-01 / MT-01 / LT-01 | Smoked walnut Brushed brass | Joinery / woodwork | Wall-hung smoked walnut vanity, 1.20 m, push-open with a concealed brass finger pull. | 1.20 m | Master Ensuite One — Joinery |
| ST-01 / MT-01 / GL-01 | Beige marble Brushed brass | Special finish / equipment | Walk-in with a single 10 mm clear glass panel on a brass channel, marble bench, brass rain head and hand shower, linear drain. | 10 mm | Master Ensuite One — Shower |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal / fittings | Brushed brass PVD throughout · wall-hung WC with concealed cistern and a brass plate. | TBC by sample/shop drawing | Master Ensuite One — Fittings |
| MT-01 / LT-01 | Brushed brass 2700K LED | Special finish / equipment | Backlit brass-framed mirror, IP-rated warm spots over the shower, a lit niche, and a 5% floor-level night circuit. | 5% | Master Ensuite One — Lighting |
| ST-01 | Beige marble | Bathroom floor | Beige marble, honed anti-slip (R10) finish. Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Master Ensuite One — full floor |
| ST-01 | Beige marble | Bathroom walls | Beige marble full-height cladding, honed. Supplier/Product Code to be selected. | 1200×600 mm · 15–20 mm | Master Ensuite One — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Master Ensuite One — non-tiled wall areas |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding behind vanity with backlit mirror. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Master Ensuite One — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Master Ensuite One — full ceiling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Vanity | Wall-hung vanity, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Master Ensuite One — vanity wall |
| ST-01 / GL-01 / LT-01 | Beige marble Clear glass | Shower area | Beige marble walls, fluted marble niche, 10 mm low-iron frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Master Ensuite One — walk-in shower |
| ST-01 | Beige marble | Countertop | Beige marble top with integrated moulded basin, honed. Supplier/Product Code to be selected. | 20 mm | Master Ensuite One — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brushed brass (pvd or solid), satin lacquered flush plate & mixers. Supplier/Product Code to be selected. | — | Master Ensuite One — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brushed brass (PVD or solid), satin lacquered — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Master Ensuite One — all fittings |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Master Ensuite One — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master Bedroom Two — South-East Suite

Upper floor  ·  6.95 m × 5.00 m (5.50 m to the terrace line) · ensuite 1.85 m × 3.05 m · direct access to the 4.90 m × 1.20 m terrace

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| TX-01 | Textiles | Special finish / equipment | 200×200 super king on the north wall, 1.20 m clear both sides, a 1.80 m bouclé bench at the foot. The window bay to the terrace is kept free for the lounge pair so the door swing and sliding track stay unobstructed. | 1.20 m · 1.80 m | Master Bedroom Two — South-East Suite — Bed & layout |
| ST-01 / MT-01 / TX-01 / LT-01 | Beige marble Brushed brass | Walls / paint | Book-matched warm beige marble, full height, with a low taupe velvet headboard and a continuous brushed brass ledge at 900 mm carrying concealed LED that grazes the stone upward. | 900 mm | Master Bedroom Two — South-East Suite — Headboard / feature wall |
| JN-01 / MT-01 / LT-01 | Smoked walnut Brushed brass | Joinery / woodwork | 4.20 m of full-height fluted walnut wardrobe with brass reveals, including a mirrored dressing bay with internal LED, jewellery drawers and a pull-out valet rail. | 4.20 m | Master Bedroom Two — South-East Suite — Wardrobes & dressing |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Walls / paint | 75" screen set into a slatted oak feature wall with warm backlighting behind the slats, over a floating oak console. | TBC by sample/shop drawing | Master Bedroom Two — South-East Suite — TV wall |
| ST-03 / MT-01 / TX-01 | Travertine Brushed brass | Special finish / equipment | Two curved cream bouclé armchairs with a round travertine table in the bay, and a taupe velvet chaise with a brass reading lamp at the terrace threshold. | TBC by sample/shop drawing | Master Bedroom Two — South-East Suite — Lounge & chaise |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Ceiling | Deep coffer with a brass reveal and 3000 K cove, trimless spots, a brass-and-glass chandelier centred on the bed, and a separate terrace-view scene that drops the interior to 15% so the outside reads. | 3000 K · 15% | Master Bedroom Two — South-East Suite — Ceiling & lighting |
| JN-02 / TX-01 | Natural oak / wood Textiles | Floor / paving | Wide-plank warm oak, 220 mm, with a 3.00×4.00 m silk-wool rug in greige under the bed and bench. | 220 mm · 4.00 m | Master Bedroom Two — South-East Suite — Flooring & rug |
| TX-01 | Textiles | Doors / window treatment | Motorised ceiling-recessed twin track across the full terrace elevation: ivory sheer plus heavy taupe blackout, both wall-to-wall. | TBC by sample/shop drawing | Master Bedroom Two — South-East Suite — Curtains |
| ST-03 / ST-01 / JN-01 / JN-02 / MT-01 / TX-01 | Travertine Beige marble | Special finish / equipment | Book-matched beige marble · taupe velvet · smoked walnut · oak · travertine · brushed brass. | TBC by sample/shop drawing | Master Bedroom Two — South-East Suite — Materials |
| JN-01 / JN-02 / LT-01 | Smoked walnut Natural oak / wood | Flooring | Engineered smoked walnut / oak plank floor, matt oiled. Supplier/Product Code to be selected. | 190×1900 mm · 15 mm | Master Bedroom Two — South-East Suite — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Bedroom Two — South-East Suite — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Bedroom Two — South-East Suite — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Master Bedroom Two — South-East Suite — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Bedroom Two — South-East Suite — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Bedroom Two — South-East Suite — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Master Bedroom Two — South-East Suite — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Bedroom Two — South-East Suite — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Bedroom Two — South-East Suite — all visible metalwork |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Wardrobes | Built-in full-height wardrobes, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Bedroom Two — South-East Suite — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master Two — Dressing Room

Upper floor  ·  3.10 m × 2.20 m · U-shaped fit-out linking bedroom and ensuite

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| SP-01 | Dark bronze / special | Special finish / equipment | U-shaped full-height joinery on three walls with a 1.60 m × 0.80 m island, 1.00 m clear on all sides. | 1.60 m × 0.80 m · 1.00 m | Master Two — Dressing Room — Layout |
| JN-01 | Smoked walnut | Joinery / woodwork | Smoked walnut carcasses with fluted and plain door mixes, double hanging, long hanging, shoe racks and a dedicated luggage bay above. | TBC by sample/shop drawing | Master Two — Dressing Room — Wardrobes |
| ST-03 / GL-01 / TX-01 | Travertine Clear glass | Countertops / stone | Travertine top with 20 mm mitred edge over six velvet-lined drawers, plus a shallow glass-topped display drawer for watches and jewellery. | 20 mm | Master Two — Dressing Room — Island |
| GL-01 / LT-01 | Clear glass 2700K LED | Special finish / equipment | Backlit reeded glass display bays, LED under every shelf and rail, and two small alabaster pendants over the island, all 3000 K and dimmed. | 3000 K | Master Two — Dressing Room — Display & lighting |
| ST-03 / JN-01 / MT-01 / TX-01 | Travertine Smoked walnut | Joinery / woodwork | 1.40 m seated vanity in walnut with a travertine top, backlit rounded brass mirror and a cream bouclé stool. | 1.40 m | Master Two — Dressing Room — Vanity |
| ST-01 | Beige marble | Flooring | Natural beige marble (Crema Marfil / Botticino type, warm cream), honed-polished satin, bookmatched in principal rooms. Supplier/Product Code to be selected. | 1200×600 mm slabs · 20 mm thick | Master Two — Dressing Room — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Two — Dressing Room — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Two — Dressing Room — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Two — Dressing Room — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Two — Dressing Room — all wall perimeters |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Two — Dressing Room — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Two — Dressing Room — all visible metalwork |
| JN-01 / MT-01 / GL-01 / LT-01 | Smoked walnut Brushed brass | Wardrobes | Full-height wardrobes, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen fronts; bronze-tint glass display doors, integrated LED, leather-look internal drawers. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Two — Dressing Room — all wardrobe walls |
| ST-01 | Beige marble | Joinery | Dressing island / vanity with beige marble (matching floor family), honed, eased 2 mm edge top and full-length mirror. Supplier/Product Code to be selected. | 20 mm top | Master Two — Dressing Room — centre / vanity wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master Ensuite Two

Upper floor  ·  1.85 m × 3.05 m · twin basins, WC and shower

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Walls / paint | Warm travertine to full height with a 900 mm dark emperador marble band at splashback level; travertine floor with a matching solid stone vanity. | 900 mm | Master Ensuite Two — Stone |
| ST-03 / LT-01 | Travertine 2700K LED | Joinery / woodwork | 2.10 m floating solid travertine vanity with twin integrated basins and an open lower shelf for rolled towels. | 2.10 m | Master Ensuite Two — Vanity |
| MT-01 / GL-01 | Brushed brass Clear glass | Special finish / equipment | Enclosure in fluted glass with a brushed champagne brass frame, stone bench, rain head plus hand shower. | TBC by sample/shop drawing | Master Ensuite Two — Shower |
| LT-01 | 2700K LED | Special finish / equipment | Two backlit rounded mirrors, a warm cove in the ceiling recess, and a discreet dimmed circuit for night use. | TBC by sample/shop drawing | Master Ensuite Two — Lighting |
| MT-01 / TX-01 / LS-01 | Brushed brass Textiles | Special finish / equipment | A round stone stool, brass towel bars, folded linen towels and a single planted vessel. | TBC by sample/shop drawing | Master Ensuite Two — Accessories |
| ST-01 | Beige marble | Bathroom floor | Beige marble, honed anti-slip (R10) finish. Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Master Ensuite Two — full floor |
| ST-01 | Beige marble | Bathroom walls | Beige marble full-height cladding, honed. Supplier/Product Code to be selected. | 1200×600 mm · 15–20 mm | Master Ensuite Two — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Master Ensuite Two — non-tiled wall areas |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding behind vanity with backlit mirror. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Master Ensuite Two — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Master Ensuite Two — full ceiling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Vanity | Wall-hung vanity, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Master Ensuite Two — vanity wall |
| ST-01 / GL-01 / LT-01 | Beige marble Clear glass | Shower area | Beige marble walls, fluted marble niche, 10 mm low-iron frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Master Ensuite Two — walk-in shower |
| ST-01 | Beige marble | Countertop | Beige marble top with integrated moulded basin, honed. Supplier/Product Code to be selected. | 20 mm | Master Ensuite Two — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brushed brass (pvd or solid), satin lacquered flush plate & mixers. Supplier/Product Code to be selected. | — | Master Ensuite Two — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brushed brass (PVD or solid), satin lacquered — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Master Ensuite Two — all fittings |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Master Ensuite Two — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Bedroom Three — North-West Twin

Upper floor  ·  4.30 m × 4.30 m · 3.50 m opening line · direct access to the 4.85 m terrace

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| SP-01 | Dark bronze / special | Special finish / equipment | Two 100×200 singles with 700 mm between them, both on one continuous headboard panel. Beds sit on the internal wall so the terrace door and the window wall stay clear. | 700 mm | Bedroom Three — North-West Twin — Bed & layout |
| JN-02 / WF-01 / TX-01 / LT-01 | Natural oak / wood Warm white paint | Walls / paint | A 3.20 m shared headboard in soft greige linen, 1.10 m high, divided by slim oak battens, with a concealed LED wash above it against the painted panelling. | 3.20 m · 1.10 m | Bedroom Three — North-West Twin — Headboard / feature wall |
| JN-02 / MT-01 / LT-01 | Natural oak / wood Brushed brass | Joinery / woodwork | 2.80 m of full-height light oak wardrobe, handleless with thin brass reveals, plus a 1.20 m integrated oak desk with a lit shelf stack and a cream leather chair. | 2.80 m · 1.20 m | Bedroom Three — North-West Twin — Wardrobes & study |
| WF-01 / LT-01 | Warm white paint 2700K LED | Walls / paint | 55" screen on a cream micro-cement panel above the desk run, cabling fully concealed. | TBC by sample/shop drawing | Bedroom Three — North-West Twin — TV wall |
| MT-01 / LT-01 | Brushed brass 2700K LED | Ceiling | Simple recessed tray with a 3000 K cove, spots on the wardrobe faces, one slim linear brass pendant between the beds and individual reading lights at each bedhead. | 3000 K | Bedroom Three — North-West Twin — Ceiling & lighting |
| JN-02 / TX-01 | Natural oak / wood Textiles | Floor / paving | Warm oak plank flooring with a soft ivory low-pile rug across the bed zone. | TBC by sample/shop drawing | Bedroom Three — North-West Twin — Flooring & rug |
| TX-01 | Textiles | Doors / window treatment | Ivory sheer with a greige blackout drape on a recessed track, sized to clear the terrace door leaf completely when stacked. | TBC by sample/shop drawing | Bedroom Three — North-West Twin — Curtains |
| JN-02 / WF-01 / MT-01 / TX-01 / LT-01 | Natural oak / wood Warm white paint | Special finish / equipment | Light oak · greige linen · cream paint · micro-cement · brushed brass · ivory wool. | TBC by sample/shop drawing | Bedroom Three — North-West Twin — Materials |
| JN-01 / JN-02 / LT-01 | Smoked walnut Natural oak / wood | Flooring | Engineered smoked walnut / oak plank floor, matt oiled. Supplier/Product Code to be selected. | 190×1900 mm · 15 mm | Bedroom Three — North-West Twin — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Bedroom Three — North-West Twin — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Bedroom Three — North-West Twin — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Bedroom Three — North-West Twin — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Bedroom Three — North-West Twin — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Bedroom Three — North-West Twin — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Bedroom Three — North-West Twin — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Bedroom Three — North-West Twin — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Bedroom Three — North-West Twin — all visible metalwork |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Wardrobes | Built-in full-height wardrobes, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Bedroom Three — North-West Twin — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Bedroom Four — North-East Guest Room

Upper floor  ·  4.95 m × 5.00 m (5.10 m structural bay) · window wall to the north-east

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-01 | Smoked walnut | Special finish / equipment | Two 105×200 singles, convertible to one 210 king by joining the bases — a practical guest arrangement. Shared walnut nightstand between, 800 mm clear to the wardrobe faces. | 210 k · 800 mm | Bedroom Four — North-East Guest Room — Bed & layout |
| WF-01 / TX-01 | Warm white paint Textiles | Walls / paint | Individual 1.60 m tall channel-tufted headboards in warm sand velvet, set against full-height fluted cream plaster panels with a painted cap rail. | 1.60 m | Bedroom Four — North-East Guest Room — Headboard / feature wall |
| JN-01 / MT-01 / LT-01 | Smoked walnut Brushed brass | Joinery / woodwork | 3.60 m of full-height fluted walnut wardrobe with brass shadow-gap pulls, internal LED, and a dedicated luggage bay for guest use. | 3.60 m | Bedroom Four — North-East Guest Room — Wardrobes & storage |
| ST-03 / MT-01 | Travertine Brushed brass | Joinery / woodwork | 1.20 m travertine-topped console at the window with a backlit rounded brass mirror and an upholstered stool, doubling as a desk. | 1.20 m | Bedroom Four — North-East Guest Room — Vanity / console |
| MT-01 / LT-01 | Brushed brass 2700K LED | Ceiling | Tray ceiling with 3000 K cove, a compact brass drum chandelier centred, brass swing-arm sconces at each bedhead on individual switching, and a low-level night light to the ensuite door. | 3000 K | Bedroom Four — North-East Guest Room — Ceiling & lighting |
| JN-02 / TX-01 | Natural oak / wood Textiles | Floor / paving | Warm oak plank with a textured camel wool rug running under both beds. | TBC by sample/shop drawing | Bedroom Four — North-East Guest Room — Flooring & rug |
| TX-01 / LT-01 | Textiles 2700K LED | Doors / window treatment | Ivory sheer with a sand-coloured blackout drape, pinch-pleat on a concealed track, full wall width. | TBC by sample/shop drawing | Bedroom Four — North-East Guest Room — Curtains |
| ST-03 / JN-01 / WF-01 / MT-01 / TX-01 | Travertine Smoked walnut | Special finish / equipment | Sand velvet · fluted cream plaster · walnut · travertine · brushed brass · camel wool. | TBC by sample/shop drawing | Bedroom Four — North-East Guest Room — Materials |
| JN-01 / JN-02 / LT-01 | Smoked walnut Natural oak / wood | Flooring | Engineered smoked walnut / oak plank floor, matt oiled. Supplier/Product Code to be selected. | 190×1900 mm · 15 mm | Bedroom Four — North-East Guest Room — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Bedroom Four — North-East Guest Room — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Bedroom Four — North-East Guest Room — all painted walls |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Bedroom Four — North-East Guest Room — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Bedroom Four — North-East Guest Room — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brushed brass (pvd or solid), satin lacquered trim / flush painted MDF | 80 mm high · 10 mm reveal | Bedroom Four — North-East Guest Room — all wall perimeters |
| JN-01 | Smoked walnut | Joinery | Smoked walnut veneer on MR-MDF, matt lacquer 10% sheen. Supplier/Product Code to be selected. | 18–22 mm core · 0.6 mm veneer | Bedroom Four — North-East Guest Room — built-in units and wall panelling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Bedroom Four — North-East Guest Room — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brushed brass (PVD or solid), satin lacquered — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Bedroom Four — North-East Guest Room — all visible metalwork |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Wardrobes | Built-in full-height wardrobes, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Bedroom Four — North-East Guest Room — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Shared Bathroom & WCs

Upper floor  ·  Two WCs at 1.80 m × 2.80 m serving the north bedrooms, exactly as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 | Travertine | Walls / paint | Cream travertine large-format wall and floor tiles, honed, with a stone-clad WC duct. | TBC by sample/shop drawing | Shared Bathroom & WCs — Stone |
| JN-02 | Natural oak / wood | Joinery / woodwork | Wall-hung stone vanity with an integrated basin and an open oak shelf below for towels and a basket. | TBC by sample/shop drawing | Shared Bathroom & WCs — Vanity |
| MT-01 / GL-01 | Brushed brass Clear glass | Special finish / equipment | Walk-in with a fixed glass panel, brass rain head and hand shower, lit niche, linear drain. | TBC by sample/shop drawing | Shared Bathroom & WCs — Shower |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal / fittings | Brushed brass tapware, wall-hung WC with a brass flush plate and concealed cistern. | TBC by sample/shop drawing | Shared Bathroom & WCs — Fittings |
| LT-01 | 2700K LED | Special finish / equipment | Slim full-height backlit mirror, warm IP spots, and a floor-level night light. | TBC by sample/shop drawing | Shared Bathroom & WCs — Lighting |
| ST-01 | Beige marble | Bathroom floor | Beige marble, honed anti-slip (R10) finish. Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Shared Bathroom & WCs — full floor |
| ST-01 | Beige marble | Bathroom walls | Beige marble full-height cladding, honed. Supplier/Product Code to be selected. | 1200×600 mm · 15–20 mm | Shared Bathroom & WCs — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Shared Bathroom & WCs — non-tiled wall areas |
| ST-02 / ST-03 / ST-01 | Dark Emperador Travertine | Feature wall | Bookmatched Silver Travertine / Dark Emperador marble slab cladding behind vanity with backlit mirror. Supplier/Product Code to be selected. | 20 mm slabs, full-height bookmatch | Shared Bathroom & WCs — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Shared Bathroom & WCs — full ceiling |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Vanity | Wall-hung vanity, smoked walnut veneer on mr-mdf, matt lacquer 10% sheen, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Shared Bathroom & WCs — vanity wall |
| ST-01 / GL-01 / LT-01 | Beige marble Clear glass | Shower area | Beige marble walls, fluted marble niche, 10 mm low-iron frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Shared Bathroom & WCs — walk-in shower |
| ST-01 | Beige marble | Countertop | Beige marble top with integrated moulded basin, honed. Supplier/Product Code to be selected. | 20 mm | Shared Bathroom & WCs — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brushed brass (pvd or solid), satin lacquered flush plate & mixers. Supplier/Product Code to be selected. | — | Shared Bathroom & WCs — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brushed brass (PVD or solid), satin lacquered — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Shared Bathroom & WCs — all fittings |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Doors | Full-height flush door, smoked walnut veneer, concealed frame and hinges, magnetic lock, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Shared Bathroom & WCs — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# North Terrace

Upper floor  ·  4.85 m × 1.80 m · off Bedroom Three

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 | Travertine | Floor / paving | 600×1200 mm warm travertine, bush-hammered for slip resistance, laid to a 1:80 fall away from the door threshold. | 1200 mm | North Terrace — Paving |
| ST-03 / JN-02 | Travertine Natural oak / wood | Special finish / equipment | Built-in teak bench, 3.20 m, with quick-dry cream outdoor cushions, and a solid travertine coffee table. | 3.20 m | North Terrace — Furniture |
| LS-01 | Planting | Special finish / equipment | Two large ribbed stone planters with standard olive trees underplanted with trailing rosemary; drip irrigation from the roof line. | TBC by sample/shop drawing | North Terrace — Planting |
| MT-01 / LT-01 | Brushed brass 2700K LED | Special finish / equipment | Concealed warm LED in the soffit and under the bench, plus one portable brass lantern. All on the terrace scene, dimmable. | TBC by sample/shop drawing | North Terrace — Lighting |
| MT-01 / GL-01 | Brushed brass Clear glass | Metal / fittings | Clear laminated glass with a brushed brass capping rail, matching the villa's metal finish. | TBC by sample/shop drawing | North Terrace — Balustrade |
| ST-04 / JN-02 | Limestone Natural oak / wood | Outdoor flooring | Bush-hammered beige limestone, R11 anti-slip with thermally modified ash / teak-tone hardwood decking zones. Supplier/Product Code to be selected. | 600×600 mm · 20 mm | North Terrace — full terrace, laid to falls |
| WF-01 | Warm white paint | Walls | Exterior textured acrylic render, warm sand / beige, matt. Supplier/Product Code to be selected. | — | North Terrace — parapet and back walls |
| MT-01 / GL-01 | Brushed brass Clear glass | Balustrade | Clear frameless laminated glass, brushed brass (pvd or solid), satin lacquered cap. Supplier/Product Code to be selected. | 12+12 mm · 1100 mm | North Terrace — open edges |
| JN-02 / WF-01 | Natural oak / wood Warm white paint | Ceiling | Exterior soffit, cement board painted warm white / wood-effect aluminium slats. Supplier/Product Code to be selected. | — | North Terrace — covered area |
| SP-01 | Dark bronze / special | Skirting | Matching paver upstand | 100 mm | North Terrace — wall base |
| MT-01 | Brushed brass | Doors | Thermally broken slim aluminium sliding doors, dark bronze, double-glazed. Supplier/Product Code to be selected. | Per elevation | North Terrace — opening to bedroom/lobby |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Dark bronze powder-coated aluminium / brushed brass (pvd or solid), satin lacquered lights. Supplier/Product Code to be selected. | — | North Terrace — fixtures |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Master Terrace

Upper floor  ·  4.90 m × 1.20 m · off Master Bedroom Two

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 | Travertine | Floor / paving | Matching bush-hammered travertine, laid in the same direction as the north terrace. | TBC by sample/shop drawing | Master Terrace — Paving |
| JN-02 / TX-01 | Natural oak / wood Textiles | Special finish / equipment | Two curved teak loungers with cream outdoor bouclé-look cushions and a round stone side table, all sized to leave a 700 mm clear walking line to the balustrade. | 700 mm | Master Terrace — Furniture |
| LS-01 | Planting | Special finish / equipment | A continuous 300 mm linear planter along the balustrade with lavender and ornamental grasses — scent and movement rather than mass. | 300 mm | Master Terrace — Planting |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Recessed soffit downlights on the door line, a concealed strip washing the planter, and wall-grazers between the glazing bays. No fixture above eye level. | TBC by sample/shop drawing | Master Terrace — Lighting |
| JN-02 | Natural oak / wood | Special finish / equipment | A slatted teak screen at the shared end gives the master terrace visual separation from the neighbouring elevation without closing the view. | TBC by sample/shop drawing | Master Terrace — Privacy |
| ST-04 / JN-02 | Limestone Natural oak / wood | Outdoor flooring | Bush-hammered beige limestone, R11 anti-slip with thermally modified ash / teak-tone hardwood decking zones. Supplier/Product Code to be selected. | 600×600 mm · 20 mm | Master Terrace — full terrace, laid to falls |
| WF-01 | Warm white paint | Walls | Exterior textured acrylic render, warm sand / beige, matt. Supplier/Product Code to be selected. | — | Master Terrace — parapet and back walls |
| MT-01 / GL-01 | Brushed brass Clear glass | Balustrade | Clear frameless laminated glass, brushed brass (pvd or solid), satin lacquered cap. Supplier/Product Code to be selected. | 12+12 mm · 1100 mm | Master Terrace — open edges |
| JN-02 / WF-01 | Natural oak / wood Warm white paint | Ceiling | Exterior soffit, cement board painted warm white / wood-effect aluminium slats. Supplier/Product Code to be selected. | — | Master Terrace — covered area |
| SP-01 | Dark bronze / special | Skirting | Matching paver upstand | 100 mm | Master Terrace — wall base |
| MT-01 | Brushed brass | Doors | Thermally broken slim aluminium sliding doors, dark bronze, double-glazed. Supplier/Product Code to be selected. | Per elevation | Master Terrace — opening to bedroom/lobby |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Dark bronze powder-coated aluminium / brushed brass (pvd or solid), satin lacquered lights. Supplier/Product Code to be selected. | — | Master Terrace — fixtures |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Front façade

Exterior & outdoor  ·  01 · Street elevation

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-04 / LT-01 | Limestone 2700K LED | Walls / paint | Large-format honed limestone in a warm ivory tone, installed with narrow expressed joints. | TBC by sample/shop drawing | Front façade — Primary finish |
| JN-01 / WF-01 | Smoked walnut Warm white paint | Special finish / equipment | Warm greige mineral render to recessed planes; smoked-walnut-effect aluminium fins at vertical accents. | TBC by sample/shop drawing | Front façade — Contrast volumes |
| MT-01 / GL-01 | Brushed brass Clear glass | Metal / fittings | Dark bronze window frames, copings and shadow-line trims; existing glass balcony retained. | TBC by sample/shop drawing | Front façade — Metalwork |
| LT-01 | 2700K LED | Special finish / equipment | Concealed linear light under the cantilever, narrow-beam façade grazers and shielded balcony downlights, all 2700 K. | 2700 K | Front façade — Lighting |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Front façade — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Front façade — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Front façade — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Front façade — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Front façade — per lighting layout |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Main entrance, gates & driveway

Exterior & outdoor  ·  02 · Arrival

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / MT-01 | Travertine Brushed brass | Special finish / equipment | Book-matched vein-cut travertine lining to walls and soffit, framed by a 20 mm champagne-bronze reveal. | 20 mm | Main entrance, gates & driveway — Portal |
| JN-01 / MT-01 / LT-01 | Smoked walnut Brushed brass | Doors / window treatment | Full-height walnut pivot door with a slim bronze pull and concealed access control. | TBC by sample/shop drawing | Main entrance, gates & driveway — Door |
| ST-03 / MT-01 | Travertine Brushed brass | Special finish / equipment | Travertine-clad solid walls with warm greige bands and dark-bronze vertical slat gates. | TBC by sample/shop drawing | Main entrance, gates & driveway — Boundary |
| SP-01 | Dark bronze / special | Floor / paving | Honed beige stone modules with charcoal basalt bands guiding cars and pedestrians independently. | TBC by sample/shop drawing | Main entrance, gates & driveway — Paving |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Main entrance, gates & driveway — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Main entrance, gates & driveway — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Main entrance, gates & driveway — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Main entrance, gates & driveway — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Main entrance, gates & driveway — per lighting layout |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Rear façade & pool terrace

Exterior & outdoor  ·  03 · Garden elevation

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / LT-01 | Travertine 2700K LED | Special finish / equipment | 11.00 × 5.92 m footprint retained; pale grey-green stone interior, flush travertine coping and submerged step lighting. | 5.92 m | Rear façade & pool terrace — Pool |
| ST-03 / LT-01 | Travertine 2700K LED | Special finish / equipment | Slip-resistant honed travertine continuing to the threshold; linear drainage concealed at the façade. | TBC by sample/shop drawing | Rear façade & pool terrace — Terrace |
| JN-01 / LT-01 | Smoked walnut 2700K LED | Special finish / equipment | Walnut-effect aluminium pergola slats with integrated dimmable linear lighting and concealed rainwater management. | TBC by sample/shop drawing | Rear façade & pool terrace — Shade |
| JN-02 | Natural oak / wood | Special finish / equipment | Low teak lounge seating, cream performance upholstery, sculptural stone tables and a separate outdoor dining setting. | TBC by sample/shop drawing | Rear façade & pool terrace — Furniture |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Rear façade & pool terrace — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Rear façade & pool terrace — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Rear façade & pool terrace — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Rear façade & pool terrace — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Rear façade & pool terrace — per lighting layout |
| ST-04 / TL-01 / GL-01 | Limestone Marble-look porcelain | Pool finish | Pool interior in sand-beige glass mosaic / porcelain, coping in honed bush-hammered beige limestone, r11 anti-slip. Supplier/Product Code to be selected. | 11.00×5.92 m pool · coping 600×400×30 mm | Rear façade & pool terrace — pool shell and coping |
| ST-04 | Limestone | Pool deck | Bush-hammered beige limestone, R11 anti-slip, anti-slip. Supplier/Product Code to be selected. | 600×600 mm · 20 mm | Rear façade & pool terrace — sun deck |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Landscape, walkways & outdoor living

Exterior & outdoor  ·  04 · External rooms

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| LS-01 | Planting | Special finish / equipment | Multi-stem olive trees, date palms where clearance permits, agave, westringia and ornamental grasses in layered beds. | TBC by sample/shop drawing | Landscape, walkways & outdoor living — Planting |
| ST-04 | Limestone | Special finish / equipment | 600 × 900 mm anti-slip limestone pavers with gravel drainage margins and flush thresholds. | 900 mm | Landscape, walkways & outdoor living — Walkways |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Low-glare spike lights to specimen trees, recessed step lights and shielded bollards along circulation routes. | TBC by sample/shop drawing | Landscape, walkways & outdoor living — Garden light |
| JN-01 / MT-01 | Smoked walnut Brushed brass | Special finish / equipment | Existing plan position retained; stone counter, walnut-effect weatherproof cabinetry and a bronze-framed pergola define the outdoor dining room. | TBC by sample/shop drawing | Landscape, walkways & outdoor living — BBQ zone |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Landscape, walkways & outdoor living — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Landscape, walkways & outdoor living — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Landscape, walkways & outdoor living — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Landscape, walkways & outdoor living — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Landscape, walkways & outdoor living — per lighting layout |
| ST-04 / WF-01 / LS-01 | Limestone Warm white paint | Softscape | Natural lawn, olive / palm feature trees, low hedges, raised planters in render / limestone, drip irrigation. Supplier/Product Code to be selected. | — | Landscape, walkways & outdoor living — planting beds and lawn |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Swimming pool & sun deck

Exterior & outdoor  ·  11.00 × 5.92 m · depth −0.50 to −1.60 m

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 | Travertine | Walls / paint | Pale grey-green honed stone mosaic interior producing a soft green-blue water colour; 400 mm flush travertine coping with a 10 mm shadow gap. | 400 mm · 10 mm | Swimming pool & sun deck — Finish |
| LT-01 | 2700K LED | Special finish / equipment | Full-width submerged entry steps at the shallow (−0.50 m) end with a 400 mm sun ledge for children and shallow seating. | 0.50 m · 400 mm | Swimming pool & sun deck — Entry |
| ST-03 / JN-02 / MT-01 | Travertine Natural oak / wood | Special finish / equipment | Six teak loungers with cream performance cushions, low travertine side tables and two bronze-framed parasols along the villa-side deck. | TBC by sample/shop drawing | Swimming pool & sun deck — Furniture |
| LT-01 | 2700K LED | Special finish / equipment | Warm 2700 K submerged niche lights on the long wall, recessed deck markers at 2.5 m centres, no glare-visible fittings from the living room. | 2700 K · 2.5 m | Swimming pool & sun deck — Lighting |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Skimmer line and equipment kept on the north edge nearest the plant space; anti-slip R11 finish to all wet paving; safety cover point discreetly recessed. | TBC by sample/shop drawing | Swimming pool & sun deck — Technical |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Swimming pool & sun deck — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Swimming pool & sun deck — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Swimming pool & sun deck — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Swimming pool & sun deck — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Swimming pool & sun deck — per lighting layout |
| ST-04 / TL-01 / GL-01 | Limestone Marble-look porcelain | Pool finish | Pool interior in sand-beige glass mosaic / porcelain, coping in honed bush-hammered beige limestone, r11 anti-slip. Supplier/Product Code to be selected. | 11.00×5.92 m pool · coping 600×400×30 mm | Swimming pool & sun deck — pool shell and coping |
| ST-04 | Limestone | Pool deck | Bush-hammered beige limestone, R11 anti-slip, anti-slip. Supplier/Product Code to be selected. | 600×600 mm · 20 mm | Swimming pool & sun deck — sun deck |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Built-in BBQ pavilion

Exterior & outdoor  ·  North-west corner · approx. 4.50 × 4.50 m

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 | Travertine | Countertops / stone | L-shaped 60 mm honed travertine worktop: 1.40 m preparation run, built-in gas grill, side burner and a raised 1.10 m bar edge with four cushioned stools. | 60 mm · 1.40 m · 1.10 m | Built-in BBQ pavilion — Counter |
| LT-01 | 2700K LED | Special finish / equipment | Undermount sink with hot and cold supply, outdoor refrigerator and ice drawer, concealed waste bin pull-out, dedicated 16 A circuits and a gas isolation valve. | 16 A | Built-in BBQ pavilion — Services |
| JN-01 | Smoked walnut | Joinery / woodwork | Walnut-effect marine-grade cabinetry with soft-close doors, a ventilated charcoal store and a lockable cleaning cupboard. | TBC by sample/shop drawing | Built-in BBQ pavilion — Storage |
| JN-02 / MT-01 / LT-01 | Natural oak / wood Brushed brass | Special finish / equipment | Dark bronze frame with wood-effect aluminium slats, concealed rainwater downpipe within one column, and dimmable warm linear light to the counter. | TBC by sample/shop drawing | Built-in BBQ pavilion — Canopy |
| ST-03 / MT-01 / LS-01 | Travertine Brushed brass | Special finish / equipment | The existing outdoor shower point on the plan is retained and finished in matching travertine with a bronze mixer, screened by planting from the cooking zone. | TBC by sample/shop drawing | Built-in BBQ pavilion — Adjacency |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Built-in BBQ pavilion — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Built-in BBQ pavilion — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Built-in BBQ pavilion — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Built-in BBQ pavilion — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Built-in BBQ pavilion — per lighting layout |
| ST-05 | Quartz | Outdoor countertop | Flamed / leathered dark granite or exterior sintered stone, UV stable. Supplier/Product Code to be selected. | 30 mm | Built-in BBQ pavilion — BBQ counter with sink & fridge |
| JN-02 / MT-01 | Natural oak / wood Brushed brass | Outdoor cabinets | Marine-grade stainless steel carcasses with dark bronze / wood-effect HPL doors. Supplier/Product Code to be selected. | — | Built-in BBQ pavilion — base units |
| JN-02 / MT-01 | Natural oak / wood Brushed brass | Canopy | Architectural canopy, dark bronze aluminium with wood-effect slat soffit. Supplier/Product Code to be selected. | Per drawing | Built-in BBQ pavilion — over BBQ |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Garden lounge & lounge deck

Exterior & outdoor  ·  North-east corner · raised deck approx. 3.06 × 3.50 m plus terrace

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-02 / TX-01 | Natural oak / wood Textiles | Special finish / equipment | Custom modular teak sofa in an L to the two closed sides, two swivel lounge chairs, cream and taupe performance fabrics with bouclé-textured scatter cushions. | TBC by sample/shop drawing | Garden lounge & lounge deck — Seating |
| ST-03 / JN-01 | Travertine Smoked walnut | Countertops / stone | Two solid travertine coffee tables at 320 mm and a pair of drum side tables in smoked walnut-effect composite. | 320 mm | Garden lounge & lounge deck — Tables |
| JN-02 / MT-01 / TX-01 / LT-01 | Natural oak / wood Brushed brass | Special finish / equipment | Bronze-framed pergola with fixed wood-effect slats angled to block midday sun; linen-look outdoor curtains on concealed tracks at the west and north edges. | TBC by sample/shop drawing | Garden lounge & lounge deck — Shade |
| LT-01 | 2700K LED | Special finish / equipment | Concealed warm cove light in the pergola beam, two dimmable pendants and floor-recessed uplights washing the columns. | TBC by sample/shop drawing | Garden lounge & lounge deck — Lighting |
| SP-01 | Dark bronze / special | Special finish / equipment | Ceiling fan integrated into the canopy, discreet weatherproof sockets and a lidded cushion store below the deck. | TBC by sample/shop drawing | Garden lounge & lounge deck — Comfort |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Garden lounge & lounge deck — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Garden lounge & lounge deck — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Garden lounge & lounge deck — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Garden lounge & lounge deck — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Garden lounge & lounge deck — per lighting layout |
| JN-02 / MT-01 / TX-01 | Natural oak / wood Brushed brass | Pergola | Dark bronze aluminium pergola with wood-effect louvres, outdoor-fabric upholstery in sand / taupe. Supplier/Product Code to be selected. | Per drawing | Garden lounge & lounge deck — seating area |
| ST-04 / WF-01 / LS-01 | Limestone Warm white paint | Softscape | Natural lawn, olive / palm feature trees, low hedges, raised planters in render / limestone, drip irrigation. Supplier/Product Code to be selected. | — | Garden lounge & lounge deck — planting beds and lawn |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Terrace dining under the pergola

Exterior & outdoor  ·  Rear terrace, aligned with the internal dining and kitchen doors

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / JN-01 | Travertine Smoked walnut | Countertops / stone | 3.00 m travertine top on a smoked walnut plinth base, seating ten in woven cream armchairs with quick-dry cushions. | 3.00 m | Terrace dining under the pergola — Table |
| JN-02 / TX-01 | Natural oak / wood Textiles | Special finish / equipment | Continuation of the wood-slat pergola with integrated retractable fabric layer for full shade in summer. | TBC by sample/shop drawing | Terrace dining under the pergola — Shade |
| LT-01 | 2700K LED | Special finish / equipment | Three woven pendants on dimmers over the table, plus wall-recessed step lights along the threshold. | TBC by sample/shop drawing | Terrace dining under the pergola — Lighting |
| ST-03 / LT-01 | Travertine 2700K LED | Special finish / equipment | A short travertine buffet ledge against the villa wall with concealed sockets, warming drawer provision and a beverage fridge. | TBC by sample/shop drawing | Terrace dining under the pergola — Service |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Terrace dining under the pergola — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Terrace dining under the pergola — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Terrace dining under the pergola — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Terrace dining under the pergola — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Terrace dining under the pergola — per lighting layout |
| JN-02 / MT-01 / TX-01 | Natural oak / wood Brushed brass | Pergola | Dark bronze aluminium pergola with wood-effect louvres, outdoor-fabric upholstery in sand / taupe. Supplier/Product Code to be selected. | Per drawing | Terrace dining under the pergola — seating area |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Children's play garden

Exterior & outdoor  ·  West lawn · within the 12.60 × 7.80 m tree area

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-02 | Natural oak / wood | Special finish / equipment | FSC oak play tower with slide, rope climb and two swings on a clean timber frame; a low balance course and a sand/water play table. | TBC by sample/shop drawing | Children's play garden — Equipment |
| LS-01 / FL-01 | Planting EPDM rubber | Floor / paving | Sand-beige wet-pour rubber with a 1.5 m critical-fall zone, laid in an organic shape and edged flush with the lawn. | 1.5 m | Children's play garden — Safety surface |
| ST-03 / LS-01 | Travertine Planting | Special finish / equipment | Curved travertine seat wall with cushions facing the play zone, in direct sight line from the living room glazing; pool access screened by a hedge and a self-closing gate. | TBC by sample/shop drawing | Children's play garden — Supervision |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Existing trees provide the primary canopy; low-level warm bollards and shielded tree uplights only, no glare at child height. | TBC by sample/shop drawing | Children's play garden — Shade & light |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Children's play garden — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Children's play garden — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Children's play garden — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Children's play garden — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Children's play garden — per lighting layout |
| FL-01 | EPDM rubber | Safety flooring | Wet-pour EPDM rubber safety surface, sand / sage colours. Supplier/Product Code to be selected. | 40–80 mm (per fall height, EN 1177) | Children's play garden — whole play zone |
| ST-04 / WF-01 / LS-01 | Limestone Warm white paint | Softscape | Natural lawn, olive / palm feature trees, low hedges, raised planters in render / limestone, drip irrigation. Supplier/Product Code to be selected. | — | Children's play garden — planting beds and lawn |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 1 · HIGH-END MODERN LUXURY

# Garden, walkways and planters

Exterior & outdoor  ·  Perimeter strips, west lawn and boundary beds per plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| LS-01 | Planting | Special finish / equipment | The three existing mature trees on the west lawn are retained and crown-lifted; date palms and multi-stem olives reinforce the boundary strips shown on the plan. | TBC by sample/shop drawing | Garden, walkways and planters — Trees |
| LS-01 | Planting | Special finish / equipment | Structure from clipped myrtle and box near the villa; texture from westringia, lavender, agave, rosemary and ornamental grasses at the perimeter. Drought-tolerant throughout. | TBC by sample/shop drawing | Garden, walkways and planters — Planting |
| LS-01 | Planting | Special finish / equipment | Warm-season turf to the west area with a flush stone mowing edge; irrigated by sub-surface drip with a smart controller. | TBC by sample/shop drawing | Garden, walkways and planters — Lawn |
| ST-04 | Limestone | Floor / paving | 600 × 900 mm anti-slip honed limestone with 10 mm joints and gravel drainage margins; charcoal basalt bands mark thresholds and changes of zone. | 900 mm · 10 mm | Garden, walkways and planters — Paving |
| ST-03 / LS-01 | Travertine Planting | Special finish / equipment | Large monolithic travertine planters at the terrace corners and pavilion entries, planted with olive standards and trailing rosemary. | TBC by sample/shop drawing | Garden, walkways and planters — Planters |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | One warm 2700 K family throughout: tree uplights, shielded 400 mm bollards on walkways, recessed step lights and grazers on the boundary wall — every source shielded from seated eye level. | 2700 K · 400 mm | Garden, walkways and planters — Lighting |
| ST-04 | Limestone | Outdoor paving | Bush-hammered beige limestone, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Garden, walkways and planters — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Garden, walkways and planters — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Garden, walkways and planters — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Thermally modified ash / teak-tone hardwood decking. Supplier/Product Code to be selected. | 140×21 mm boards | Garden, walkways and planters — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Garden, walkways and planters — per lighting layout |
| ST-04 / WF-01 / LS-01 | Limestone Warm white paint | Softscape | Natural lawn, olive / palm feature trees, low hedges, raised planters in render / limestone, drip irrigation. Supplier/Product Code to be selected. | — | Garden, walkways and planters — planting beds and lawn |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Main Entrance & Lobby

Ground floor  ·  Entry axis 4.20 m deep · 1.30 m opening to the living wing

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Floor / paving | 120×120 cm marble-look porcelain, rectified, straight set with 2 mm joints. No inlay medallion, no waterjet cutting — the single biggest saving on this floor. | 120 cm · 2 mm | Main Entrance & Lobby — Flooring |
| JN-02 / WF-01 | Natural oak / wood Warm white paint | Walls / paint | Premium washable matt paint in warm greige, with one feature wall in a factory-made oak-veneer slat panel system instead of carved fluted stone. | TBC by sample/shop drawing | Main Entrance & Lobby — Walls |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Ceiling | Simple single-step gypsum drop with a continuous cove (3000 K) and standard recessed spots. One catalogue glass-and-black-metal pendant replaces the bespoke brass cascade. | 3000 K | Main Entrance & Lobby — Ceiling & lighting |
| ST-01 / JN-02 | Beige marble Natural oak / wood | Joinery / woodwork | Modular flush cloak cupboard in melamine carcass with veneer fronts; wall-hung shelf console in the same veneer instead of a solid marble slab. | TBC by sample/shop drawing | Main Entrance & Lobby — Joinery |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Main Entrance & Lobby — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Main Entrance & Lobby — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Main Entrance & Lobby — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Main Entrance & Lobby — entrance feature wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Main Entrance & Lobby — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Main Entrance & Lobby — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Main Entrance & Lobby — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Main Entrance & Lobby — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Main Entrance & Lobby — all visible metalwork |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Formal Guest Lounge

Ground floor  ·  9.50 m long × 5.65 m deep — south wing, direct off the entrance

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 / JN-02 / MT-01 | Beige marble Marble-look porcelain | Special finish / equipment | 1.60 m linear electric/gas insert in a marble-look porcelain slab surround with an oak-veneer reveal instead of honed emperador and brass. Same visual weight, a fraction of the stone cost. | 1.60 m | Formal Guest Lounge — Fireplace |
| TL-01 / WF-01 / MT-01 / TX-01 | Marble-look porcelain Warm white paint | Walls / paint | Porcelain slab centre panel flanked by painted MDF grooved panelling in warm sand — the upholstered fabric panels and brass profiles of Option 1 are dropped. | 1 a | Formal Guest Lounge — Feature wall |
| ST-01 / TX-01 | Beige marble Textiles | Special finish / equipment | Locally made perimeter sofas, 90 cm deep, in a heavy performance weave rather than velvet; two ceramic-top side tables replacing solid marble cubes. | 90 cm | Formal Guest Lounge — Furniture |
| LT-01 | 2700K LED | Ceiling | Single-step perimeter cove, no stepped profile. Two medium catalogue pendants instead of one large bespoke chandelier, plus dimmable spots. | TBC by sample/shop drawing | Formal Guest Lounge — Ceiling & lighting |
| TX-01 | Textiles | Doors / window treatment | Sheer plus lined cream drapery on a recessed track — the one detail kept exactly as Option 1. | TBC by sample/shop drawing | Formal Guest Lounge — Curtains |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Formal Guest Lounge — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Formal Guest Lounge — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Formal Guest Lounge — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Formal Guest Lounge — fireplace / TV wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Formal Guest Lounge — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Formal Guest Lounge — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Formal Guest Lounge — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Formal Guest Lounge — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Formal Guest Lounge — all visible metalwork |
| ST-03 / TL-01 / MT-01 | Travertine Marble-look porcelain | Fireplace | Linear built-in fireplace with large-format travertine-look porcelain slab cladding (stone-effect) surround and brass-effect brushed pvd / powder-coat champagne bronze trim. Supplier/Product Code to be selected. | 1500–1800 mm firebox | Formal Guest Lounge — fireplace wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Main Family Living Room

Ground floor  ·  9.41 × 6.90 m — north-west, open to the kitchen

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Walls / paint | Two 160×320 cm marble-look porcelain slabs (not book-matched natural marble) with a recessed screen niche and a 1.80 m linear fire beneath, in one plane. | 320 cm · 1.80 m | Main Family Living Room — Feature wall |
| JN-01 / JN-02 / LT-01 | Smoked walnut Natural oak / wood | Joinery / woodwork | 3.60 m floating console in oak-effect laminate with a veneer face and LED shadow gap — visually identical at 2 m, roughly a third of the smoked-walnut cost. | 3.60 m · 2 m | Main Family Living Room — Joinery |
| JN-02 / TX-01 | Natural oak / wood Textiles | Walls / paint | Prefabricated acoustic slat panels on the long internal wall instead of site-built oak slats on acoustic backing. | TBC by sample/shop drawing | Main Family Living Room — Accent wall |
| LT-01 | 2700K LED | Ceiling | Flat ceiling with a single perimeter cove and a linear magnetic track over each cluster; one modest decorative fixture per bay. | TBC by sample/shop drawing | Main Family Living Room — Ceiling & lighting |
| TX-01 | Textiles | Special finish / equipment | Cream weave four-seater, four fabric lounge chairs, ceramic-top coffee tables; second bay with two locally made sofas. | TBC by sample/shop drawing | Main Family Living Room — Furniture |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Main Family Living Room — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Main Family Living Room — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Main Family Living Room — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Main Family Living Room — fireplace / TV wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Main Family Living Room — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Main Family Living Room — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Main Family Living Room — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Main Family Living Room — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Main Family Living Room — all visible metalwork |
| ST-03 / TL-01 / MT-01 | Travertine Marble-look porcelain | Fireplace | Linear built-in fireplace with large-format travertine-look porcelain slab cladding (stone-effect) surround and brass-effect brushed pvd / powder-coat champagne bronze trim. Supplier/Product Code to be selected. | 1500–1800 mm firebox | Main Family Living Room — fireplace wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Dining Area

Ground floor  ·  5.65 m deep × 4.20 m — north-east of the guest lounge, as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-05 / JN-01 / TX-01 | Quartz Smoked walnut | Countertops / stone | 1.80 m round table with a sintered-stone top on a veneered MDF pedestal; eight upholstered fabric chairs from a local maker instead of leather and walnut. | 1.80 m | Dining Area — Table & seating |
| TL-01 / JN-02 | Marble-look porcelain Natural oak / wood | Joinery / woodwork | Low buffet only (no full-height crockery unit), oak-veneer fronts with a porcelain top and a single lit open niche above. | TBC by sample/shop drawing | Dining Area — Joinery |
| MT-01 / GL-01 / LT-01 | Brushed brass Clear glass | Special finish / equipment | One three-tier catalogue pendant in matt bronze with opal glass on a dimmer. | TBC by sample/shop drawing | Dining Area — Lighting |
| WF-01 / MT-01 | Warm white paint Brushed brass | Walls / paint | Premium paint with one textured wallpaper panel replacing the antique-bronze mirror wall. | TBC by sample/shop drawing | Dining Area — Walls |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Dining Area — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Dining Area — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Dining Area — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Dining Area — dining feature wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Dining Area — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Dining Area — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Dining Area — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Dining Area — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Dining Area — all visible metalwork |
| ST-05 / JN-01 / JN-02 | Quartz Smoked walnut | Countertops / stone | Dining table top in quartz, warm white with soft grey vein, polished; walnut/oak sideboard. Supplier/Product Code to be selected. | 20–30 mm | Dining Area — table & sideboard |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Kitchen

Ground floor  ·  North band of the 12.00 m frontage, open to the living and dining areas, service door to the maid's room

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-02 | Natural oak / wood | Joinery / woodwork | High-quality modular handleless system: matt cream lacquer-effect tall run, oak-effect melamine island. Appliances integrated except a single visible oven column. | TBC by sample/shop drawing | Kitchen — Cabinetry |
| TL-01 / ST-05 | Marble-look porcelain Quartz | Floor / paving | Engineered quartz worktops and a 60 cm porcelain splashback rather than a full-height book-matched slab. | 60 cm | Kitchen — Surfaces |
| SP-01 | Dark bronze / special | Countertops / stone | Straight square edge instead of a waterfall return, 30 cm overhang, three stools, surface-mounted socket strip. | 30 cm | Kitchen — Island |
| GL-01 / LT-01 | Clear glass 2700K LED | Special finish / equipment | Three simple opal-glass pendants and under-cabinet LED, all 3000 K. | 3000 K | Kitchen — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Kitchen — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Kitchen — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Kitchen — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Kitchen — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Kitchen — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Kitchen — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Kitchen — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Kitchen — all visible metalwork |
| JN-02 | Natural oak / wood | Kitchen cabinets | Warm taupe matt lacquer / anti-fingerprint laminate base units + oak-effect tall units, handleless profile, soft-close hinges & drawers. Supplier/Product Code to be selected. | 18 mm carcass · 600 mm base depth | Kitchen — base, wall, tall units and island |
| ST-05 | Quartz | Kitchen countertop | Quartz, warm white with soft grey vein, polished. Supplier/Product Code to be selected. | 20 mm + 40 mm mitred edge / waterfall island | Kitchen — worktops and island |
| ST-05 | Quartz | Backsplash | Full-height slab in matching quartz. Supplier/Product Code to be selected. | Slab to underside of wall units | Kitchen — cooking and sink walls |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Staircase

Ground floor  ·  2.15 m wide flight · 20 risers · 2.81 m landing

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 / MT-01 | Beige marble Marble-look porcelain | Floor / paving | Marble-look large-format porcelain stair tiles with a matching bullnose nosing; matte cream risers. No brass inlay. | TBC by sample/shop drawing | Staircase — Treads & risers |
| JN-02 / GL-01 | Natural oak / wood Clear glass | Metal / fittings | Frameless clear toughened glass in a satin aluminium base channel with an oak-effect cap handrail — the same glass concept as Option 1, in a standard system rather than a bespoke one. | TBC by sample/shop drawing | Staircase — Balustrade |
| LT-01 | 2700K LED | Special finish / equipment | Handrail LED plus one linear multi-drop pendant in the void — one fixture, not a bespoke cascade. | TBC by sample/shop drawing | Staircase — Lighting |
| WF-01 | Warm white paint | Walls / paint | Textured paint finish with one large print; understair volume in modular flush storage. | TBC by sample/shop drawing | Staircase — Wall & understair |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Staircase treads | Marble-look porcelain treads with bullnose nosing, matt anti-slip. Supplier/Product Code to be selected. | Tread 300 mm · riser 160–170 mm · 20–30 mm | Staircase — all flights and landings |
| GL-01 | Clear glass | Balustrade | Clear frameless laminated toughened glass in recessed floor channel, no handrail rods. Supplier/Product Code to be selected. | 12+12 mm laminated · 1100 mm high | Staircase — open side of flights and landing |
| MT-01 | Brushed brass | Handrail | Brass-effect brushed PVD / powder-coat champagne bronze slim wall-mounted handrail. Supplier/Product Code to be selected. | Ø40 mm | Staircase — wall side |
| LT-01 | 2700K LED | Stair lighting | Recessed 2700K LED step lights / under-tread LED strip. Supplier/Product Code to be selected. | — | Staircase — every 2nd step |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish | — | Staircase — stairwell walls |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Staircase — stairwell walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Staircase — double-height stair wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights; feature pendant to double-height void. Supplier/Product Code to be selected. | — | Staircase — soffit and void |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Skirting | Stringer skirting in matching marble-look porcelain treads with bullnose nosing | 100 mm | Staircase — wall stringer |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Guest WCs

Ground floor  ·  Guest WC 2.40 × 1.20 m off the lobby · second WC 1.82 m wide on the service side

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / ST-01 / TL-01 | Travertine Beige marble | Walls / paint | 120×280 cm marble-look and travertine-look porcelain slabs on the feature wall, large-format tile elsewhere. No natural stone. | 280 cm | Guest WCs — Finishes |
| SP-01 | Dark bronze / special | Special finish / equipment | Standard-size wall-hung units with veneer fronts and integrated ceramic basins rather than carved stone basins. | TBC by sample/shop drawing | Guest WCs — Vanities |
| LT-01 | 2700K LED | Special finish / equipment | Backlit mirror plus a single wall light; the lit accessory niche is retained in the guest WC only. | TBC by sample/shop drawing | Guest WCs — Lighting |
| SP-01 | Dark bronze / special | Metal / fittings | Matt black or brushed-nickel tapware from a mid-market premium range. | TBC by sample/shop drawing | Guest WCs — Fittings |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom floor | Marble-look porcelain, matt anti-slip (R10). Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Guest WCs — full floor |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom walls | Marble-look porcelain, satin. Supplier/Product Code to be selected. | 1200×600 mm · 9 mm | Guest WCs — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Guest WCs — non-tiled wall areas |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect) behind vanity with backlit mirror. Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Guest WCs — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Guest WCs — full ceiling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Vanity | Wall-hung vanity, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Guest WCs — vanity wall |
| ST-05 | Quartz | Countertop | Quartz top with undermount ceramic basin, polished. Supplier/Product Code to be selected. | 20 mm | Guest WCs — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brass-effect brushed pvd / powder-coat champagne bronze flush plate & mixers. Supplier/Product Code to be selected. | — | Guest WCs — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Guest WCs — all fittings |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Guest WCs — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Corridors & Transitional Areas

Ground floor  ·  Lobby-to-wing links and the service passage

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| TL-01 | Marble-look porcelain | Floor / paving | Continuous 120×120 cm porcelain, no inlay strip. | 120 cm | Corridors & Transitional Areas — Flooring |
| WF-01 | Warm white paint | Doors / window treatment | Standard-height veneered flush doors with simple black lever handles; service doors painted flush to the wall. | TBC by sample/shop drawing | Corridors & Transitional Areas — Doors |
| LT-01 | 2700K LED | Special finish / equipment | One cove line, spots only where a door needs marking. | TBC by sample/shop drawing | Corridors & Transitional Areas — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Corridors & Transitional Areas — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Corridors & Transitional Areas — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Corridors & Transitional Areas — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Corridors & Transitional Areas — end wall of corridor |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Corridors & Transitional Areas — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Corridors & Transitional Areas — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Corridors & Transitional Areas — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Corridors & Transitional Areas — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Corridors & Transitional Areas — all visible metalwork |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Maid's Room, Service WC & Boiler

Ground floor  ·  Maid's room on the north-east corner as drawn · service WC 1.82 m · boiler recess off the stair core

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-02 | Natural oak / wood | Special finish / equipment | Single bed, oak-effect melamine wardrobe with an open bay, laminate desk, roller blind on the drawn window. | TBC by sample/shop drawing | Maid's Room, Service WC & Boiler — Maid's room |
| TL-01 / LT-01 | Marble-look porcelain 2700K LED | Special finish / equipment | Large-format porcelain to 2.10 m, close-coupled WC, compact vanity — plumbing on the drawn wet wall. | 2.10 m | Maid's Room, Service WC & Boiler — Service WC |
| WF-01 | Warm white paint | Special finish / equipment | Washer and dryer in a ventilated niche behind flush painted doors; boiler recess retained with a louvred panel. | TBC by sample/shop drawing | Maid's Room, Service WC & Boiler — Laundry & boiler |
| TL-01 / WF-01 / LT-01 | Marble-look porcelain Warm white paint | Walls / paint | Washable cream paint, matte beige porcelain floor, recessed spots at 3000 K. | 3000 K | Maid's Room, Service WC & Boiler — Finishes & lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Maid's Room, Service WC & Boiler — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Maid's Room, Service WC & Boiler — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Maid's Room, Service WC & Boiler — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Maid's Room, Service WC & Boiler — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Maid's Room, Service WC & Boiler — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Maid's Room, Service WC & Boiler — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler — all visible metalwork |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom floor | Marble-look porcelain, matt anti-slip (R10). Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Maid's Room, Service WC & Boiler (service WC) — full floor |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom walls | Marble-look porcelain, satin. Supplier/Product Code to be selected. | 1200×600 mm · 9 mm | Maid's Room, Service WC & Boiler (service WC) — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler (service WC) — non-tiled wall areas |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect) behind vanity with backlit mirror. Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Maid's Room, Service WC & Boiler (service WC) — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Maid's Room, Service WC & Boiler (service WC) — full ceiling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Vanity | Wall-hung vanity, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Maid's Room, Service WC & Boiler (service WC) — vanity wall |
| ST-01 / TL-01 / GL-01 / LT-01 | Beige marble Marble-look porcelain | Shower area | Marble-look porcelain walls, recessed niche, 8 mm clear tempered frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Maid's Room, Service WC & Boiler (service WC) — walk-in shower |
| ST-05 | Quartz | Countertop | Quartz top with undermount ceramic basin, polished. Supplier/Product Code to be selected. | 20 mm | Maid's Room, Service WC & Boiler (service WC) — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brass-effect brushed pvd / powder-coat champagne bronze flush plate & mixers. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler (service WC) — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Maid's Room, Service WC & Boiler (service WC) — all fittings |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Maid's Room, Service WC & Boiler (service WC) — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Central Lobby & Preparation Pantry

Upper floor  ·  3.75 × 2.50 m lobby · 1.60 m pantry run on the lobby wall

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-02 | Natural oak / wood | Joinery / woodwork | 1.60 m run: modular lower cabinets with drawers, upper cabinets, and an open niche between. Beige-taupe matt fronts with an oak-effect accent, soft-close hardware. | 1.60 m | Central Lobby & Preparation Pantry — Pantry joinery |
| ST-05 | Quartz | Countertops / stone | Engineered quartz top with a 400 mm undermount stainless sink and a black mixer tap. Water and waste run in the adjacent bathroom wall — no new wet wall. | 400 mm | Central Lobby & Preparation Pantry — Countertop & sink |
| SP-01 | Dark bronze / special | Special finish / equipment | Two twin sockets above the counter for a kettle and coffee machine, one inside the niche, dedicated circuit and a local isolator. | TBC by sample/shop drawing | Central Lobby & Preparation Pantry — Electrical |
| LT-01 | 2700K LED | Special finish / equipment | Under-cabinet LED strip plus a warm niche strip on a night circuit. | TBC by sample/shop drawing | Central Lobby & Preparation Pantry — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Central Lobby & Preparation Pantry — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Central Lobby & Preparation Pantry — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Central Lobby & Preparation Pantry — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Central Lobby & Preparation Pantry — pantry wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Central Lobby & Preparation Pantry — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Central Lobby & Preparation Pantry — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Central Lobby & Preparation Pantry — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Central Lobby & Preparation Pantry — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Central Lobby & Preparation Pantry — all visible metalwork |
| JN-02 | Natural oak / wood | Kitchen cabinets | Built-in pantry, warm beige/taupe matt lacquer lower & upper units with engineered natural oak veneer / oak-effect hpl on mr-mdf, matt open niche. Supplier/Product Code to be selected. | 600 / 350 mm deep | Central Lobby & Preparation Pantry — pantry wall |
| ST-05 | Quartz | Kitchen countertop | Quartz, warm white with soft grey vein, polished with small undermount sink. Supplier/Product Code to be selected. | 20 mm | Central Lobby & Preparation Pantry — pantry |
| LT-01 | 2700K LED | Backsplash | Matching stone slab with under-cabinet LED. Supplier/Product Code to be selected. | Full height between units | Central Lobby & Preparation Pantry — pantry |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Master Suite One — Bedroom

Upper floor  ·  4.05 × 5.40 m (4.10 m at the bed wall)

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| JN-01 / TX-01 | Smoked walnut Textiles | Walls / paint | Prefabricated upholstered panel run in warm sand fabric, flanked by veneer-effect panels — no site-built walnut frame. | TBC by sample/shop drawing | Master Suite One — Bedroom — Headboard wall |
| LT-01 | 2700K LED | Ceiling | Single-step cove with spots; two catalogue pendants at the bedsides instead of a chandelier. | TBC by sample/shop drawing | Master Suite One — Bedroom — Ceiling & lighting |
| JN-02 / WF-01 | Natural oak / wood Warm white paint | Floor / paving | Premium paint plus one textured wallpaper wall; engineered oak-effect laminate flooring rather than solid or engineered oak. | TBC by sample/shop drawing | Master Suite One — Bedroom — Walls & floor |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Master Suite One — Bedroom — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Suite One — Bedroom — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Suite One — Bedroom — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Master Suite One — Bedroom — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Suite One — Bedroom — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Suite One — Bedroom — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Master Suite One — Bedroom — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Suite One — Bedroom — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Suite One — Bedroom — all visible metalwork |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Wardrobes | Built-in full-height wardrobes, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Suite One — Bedroom — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Master Suite One — Dressing

Upper floor  ·  Walk-in dressing bay off the bedroom, as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| LT-01 | 2700K LED | Joinery / woodwork | Modular full-height units, matt beige fronts, soft-close drawers, hanging double rails, two open display sections with LED strips. | TBC by sample/shop drawing | Master Suite One — Dressing — Wardrobes |
| ST-05 | Quartz | Joinery / woodwork | Wall-hung 1.10 m veneer vanity with a quartz top, drawer bank and a simple round lit mirror. | 1.10 m | Master Suite One — Dressing — Vanity |
| LT-01 | 2700K LED | Special finish / equipment | In-cabinet LED strips on door sensors, plus one flush ceiling fixture. | TBC by sample/shop drawing | Master Suite One — Dressing — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Master Suite One — Dressing — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Suite One — Dressing — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Suite One — Dressing — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Suite One — Dressing — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Suite One — Dressing — all wall perimeters |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Suite One — Dressing — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Suite One — Dressing — all visible metalwork |
| JN-02 / MT-01 / GL-01 / LT-01 | Natural oak / wood Brushed brass | Wardrobes | Full-height wardrobes, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt fronts; bronze-tint glass display doors, integrated LED, leather-look internal drawers. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Suite One — Dressing — all wardrobe walls |
| ST-05 | Quartz | Joinery | Dressing island / vanity with quartz, warm white with soft grey vein, polished top and full-length mirror. Supplier/Product Code to be selected. | 20 mm top | Master Suite One — Dressing — centre / vanity wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Master Suite One — Ensuite

Upper floor  ·  Position and plumbing walls exactly as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Floor / paving | 120×280 cm marble-look porcelain on the vanity and shower walls, matching large-format tile elsewhere. | 280 cm | Master Suite One — Ensuite — Walls & floor |
| ST-05 | Quartz | Joinery / woodwork | 1.40 m wall-hung veneer vanity, quartz top with an integrated basin, backlit mirror cabinet. | 1.40 m | Master Suite One — Ensuite — Vanity |
| GL-01 / LT-01 | Clear glass 2700K LED | Special finish / equipment | Walk-in shower with a frameless fixed glass panel, linear drain, thermostatic mixer; wall-hung WC with a concealed cistern. | TBC by sample/shop drawing | Master Suite One — Ensuite — Shower & WC |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom floor | Marble-look porcelain, matt anti-slip (R10). Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Master Suite One — Ensuite — full floor |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom walls | Marble-look porcelain, satin. Supplier/Product Code to be selected. | 1200×600 mm · 9 mm | Master Suite One — Ensuite — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Master Suite One — Ensuite — non-tiled wall areas |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect) behind vanity with backlit mirror. Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Master Suite One — Ensuite — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Master Suite One — Ensuite — full ceiling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Vanity | Wall-hung vanity, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Master Suite One — Ensuite — vanity wall |
| ST-01 / TL-01 / GL-01 / LT-01 | Beige marble Marble-look porcelain | Shower area | Marble-look porcelain walls, recessed niche, 8 mm clear tempered frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Master Suite One — Ensuite — walk-in shower |
| ST-05 | Quartz | Countertop | Quartz top with undermount ceramic basin, polished. Supplier/Product Code to be selected. | 20 mm | Master Suite One — Ensuite — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brass-effect brushed pvd / powder-coat champagne bronze flush plate & mixers. Supplier/Product Code to be selected. | — | Master Suite One — Ensuite — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Master Suite One — Ensuite — all fittings |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Master Suite One — Ensuite — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Master Suite Two — Bedroom

Upper floor  ·  6.95 × 5.00 m (5.50 m to the terrace line), with the 4.90 × 1.20 m terrace

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 / JN-02 | Beige marble Marble-look porcelain | Walls / paint | Two marble-look porcelain slabs with an oak-veneer frame, replacing book-matched natural stone. | TBC by sample/shop drawing | Master Suite Two — Bedroom — Bedhead wall |
| LT-01 | 2700K LED | Ceiling | Double cove, no coffers; one medium decorative fixture plus bedside wall lights. | TBC by sample/shop drawing | Master Suite Two — Bedroom — Ceiling & lighting |
| TX-01 | Textiles | Special finish / equipment | Upholstered bed and bench in performance fabric from a local maker; laminate-topped bedside tables with veneer fronts. | TBC by sample/shop drawing | Master Suite Two — Bedroom — Furniture |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Master Suite Two — Bedroom — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Suite Two — Bedroom — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Suite Two — Bedroom — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Master Suite Two — Bedroom — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Suite Two — Bedroom — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Suite Two — Bedroom — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Master Suite Two — Bedroom — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Suite Two — Bedroom — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Suite Two — Bedroom — all visible metalwork |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Wardrobes | Built-in full-height wardrobes, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Suite Two — Bedroom — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Master Suite Two — Dressing

Upper floor  ·  Dressing area as drawn between bedroom and ensuite

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| SP-01 | Dark bronze / special | Joinery / woodwork | Modular units on three walls, matt taupe fronts, mix of hanging, drawers and two lit open bays. | TBC by sample/shop drawing | Master Suite Two — Dressing — Wardrobes |
| ST-05 | Quartz | Special finish / equipment | Freestanding 1.20 m drawer island with a quartz top instead of a bespoke fitted island. | 1.20 m | Master Suite Two — Dressing — Central unit |
| LT-01 | 2700K LED | Special finish / equipment | Cabinet LED plus two flush ceiling fixtures. | TBC by sample/shop drawing | Master Suite Two — Dressing — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Master Suite Two — Dressing — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Master Suite Two — Dressing — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Master Suite Two — Dressing — all painted walls |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Master Suite Two — Dressing — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Master Suite Two — Dressing — all wall perimeters |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Master Suite Two — Dressing — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Master Suite Two — Dressing — all visible metalwork |
| JN-02 / MT-01 / GL-01 / LT-01 | Natural oak / wood Brushed brass | Wardrobes | Full-height wardrobes, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt fronts; bronze-tint glass display doors, integrated LED, leather-look internal drawers. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Master Suite Two — Dressing — all wardrobe walls |
| ST-05 | Quartz | Joinery | Dressing island / vanity with quartz, warm white with soft grey vein, polished top and full-length mirror. Supplier/Product Code to be selected. | 20 mm top | Master Suite Two — Dressing — centre / vanity wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Master Suite Two — Ensuite

Upper floor  ·  Position and plumbing walls exactly as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Floor / paving | Travertine-look large-format porcelain throughout, one darker feature slab behind the vanity. | TBC by sample/shop drawing | Master Suite Two — Ensuite — Walls & floor |
| ST-05 | Quartz | Joinery / woodwork | 1.80 m wall-hung double vanity, veneer fronts, quartz top with two integrated basins, full-width backlit mirror. | 1.80 m | Master Suite Two — Ensuite — Vanity |
| GL-01 / LT-01 | Clear glass 2700K LED | Special finish / equipment | Walk-in shower with fixed glass, ceiling rain head and hand shower; wall-hung WC with concealed cistern. | TBC by sample/shop drawing | Master Suite Two — Ensuite — Shower & WC |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom floor | Marble-look porcelain, matt anti-slip (R10). Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Master Suite Two — Ensuite — full floor |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom walls | Marble-look porcelain, satin. Supplier/Product Code to be selected. | 1200×600 mm · 9 mm | Master Suite Two — Ensuite — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Master Suite Two — Ensuite — non-tiled wall areas |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect) behind vanity with backlit mirror. Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Master Suite Two — Ensuite — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Master Suite Two — Ensuite — full ceiling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Vanity | Wall-hung vanity, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Master Suite Two — Ensuite — vanity wall |
| ST-01 / TL-01 / GL-01 / LT-01 | Beige marble Marble-look porcelain | Shower area | Marble-look porcelain walls, recessed niche, 8 mm clear tempered frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Master Suite Two — Ensuite — walk-in shower |
| ST-05 | Quartz | Countertop | Quartz top with undermount ceramic basin, polished. Supplier/Product Code to be selected. | 20 mm | Master Suite Two — Ensuite — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brass-effect brushed pvd / powder-coat champagne bronze flush plate & mixers. Supplier/Product Code to be selected. | — | Master Suite Two — Ensuite — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Master Suite Two — Ensuite — all fittings |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Master Suite Two — Ensuite — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Bedroom Three

Upper floor  ·  4.30 × 4.30 m

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| WF-01 | Warm white paint | Walls / paint | Premium paint with a grooved painted MDF panel behind the bed. | TBC by sample/shop drawing | Bedroom Three — Walls |
| LT-01 | 2700K LED | Joinery / woodwork | Modular wardrobe run and a laminate study desk with a veneer face and LED strip. | TBC by sample/shop drawing | Bedroom Three — Joinery |
| LT-01 | 2700K LED | Special finish / equipment | Cove plus spots; two simple wall reading lights. | TBC by sample/shop drawing | Bedroom Three — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Bedroom Three — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Bedroom Three — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Bedroom Three — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Bedroom Three — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Bedroom Three — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Bedroom Three — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Bedroom Three — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Bedroom Three — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Bedroom Three — all visible metalwork |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Wardrobes | Built-in full-height wardrobes, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Bedroom Three — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Bedroom Four

Upper floor  ·  4.95 × 5.00 m (5.10 m structural bay)

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| WF-01 | Warm white paint | Walls / paint | Textured paint or heavy wallpaper on the bed wall instead of fluted plaster. | TBC by sample/shop drawing | Bedroom Four — Walls |
| TX-01 | Textiles | Special finish / equipment | Upholstered bed in sand weave, veneer bedside tables, a fabric bench. | TBC by sample/shop drawing | Bedroom Four — Furniture |
| LT-01 | 2700K LED | Special finish / equipment | Cove, spots and two pendant drops at the bedsides. | TBC by sample/shop drawing | Bedroom Four — Lighting |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Flooring | Marble-look rectified porcelain (warm beige / cream vein), matt-satin. Supplier/Product Code to be selected. | 1200×600 mm · 9–10 mm thick | Bedroom Four — full floor area |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish ready for paint | Plaster 10–13 mm | Bedroom Four — all walls except feature wall |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Bedroom Four — all painted walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Bedroom Four — upholstered / timber-panelled headboard wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights. Supplier/Product Code to be selected. | 12.5 mm board · 2.9–3.0 m FFL | Bedroom Four — full ceiling |
| WF-01 / MT-01 | Warm white paint Brushed brass | Skirting | Recessed shadow-gap skirting, brass-effect brushed pvd / powder-coat champagne bronze trim / flush painted MDF | 80 mm high · 10 mm reveal | Bedroom Four — all wall perimeters |
| JN-02 | Natural oak / wood | Joinery | Engineered natural oak veneer / oak-effect HPL on MR-MDF, matt. Supplier/Product Code to be selected. | 18 mm core | Bedroom Four — built-in units and wall panelling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges. Supplier/Product Code to be selected. | 900×2700 mm · 50 mm leaf | Bedroom Four — entry door(s) |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — door handles, inlays, light fittings, joinery pulls. Supplier/Product Code to be selected. | — | Bedroom Four — all visible metalwork |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Wardrobes | Built-in full-height wardrobes, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, internal LED, soft-close. Supplier/Product Code to be selected. | 600 mm deep · floor to ceiling | Bedroom Four — wardrobe wall |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Terraces

Upper floor  ·  Two terraces, 4.85 m and 4.90 m long, depths as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Floor / paving | 20 mm travertine-look outdoor porcelain on pedestals rather than natural stone slabs. | 20 mm | Terraces — Paving |
| TL-01 / WF-01 / GL-01 | Marble-look porcelain Warm white paint | Metal / fittings | Rendered and painted parapet with a porcelain coping; no metal or glass balustrade added. | TBC by sample/shop drawing | Terraces — Balustrade & parapet |
| GL-01 / LT-01 / LS-01 | Clear glass 2700K LED | Special finish / equipment | Aluminium-and-rope lounge set, fibreglass planters, recessed step lights and one wall wash per terrace. | TBC by sample/shop drawing | Terraces — Furniture & lighting |
| ST-04 / TL-01 / JN-02 | Limestone Marble-look porcelain | Outdoor flooring | Limestone-look exterior porcelain, R11 anti-slip with composite wood-effect decking, teak tone zones. Supplier/Product Code to be selected. | 600×600 mm · 20 mm | Terraces — full terrace, laid to falls |
| WF-01 | Warm white paint | Walls | Exterior textured acrylic render, warm sand / beige, matt. Supplier/Product Code to be selected. | — | Terraces — parapet and back walls |
| MT-01 / GL-01 | Brushed brass Clear glass | Balustrade | Clear frameless laminated glass, brass-effect brushed pvd / powder-coat champagne bronze cap. Supplier/Product Code to be selected. | 12+12 mm · 1100 mm | Terraces — open edges |
| JN-02 / WF-01 | Natural oak / wood Warm white paint | Ceiling | Exterior soffit, cement board painted warm white / wood-effect aluminium slats. Supplier/Product Code to be selected. | — | Terraces — covered area |
| SP-01 | Dark bronze / special | Skirting | Matching paver upstand | 100 mm | Terraces — wall base |
| MT-01 | Brushed brass | Doors | Thermally broken slim aluminium sliding doors, dark bronze, double-glazed. Supplier/Product Code to be selected. | Per elevation | Terraces — opening to bedroom/lobby |
| MT-01 / LT-01 | Brushed brass 2700K LED | Metal finishes | Dark bronze powder-coated aluminium / brass-effect brushed pvd / powder-coat champagne bronze lights. Supplier/Product Code to be selected. | — | Terraces — fixtures |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Staircase Landing & Feature Wall

Upper floor  ·  Per approved upper floor plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Staircase treads | Marble-look porcelain treads with bullnose nosing, matt anti-slip. Supplier/Product Code to be selected. | Tread 300 mm · riser 160–170 mm · 20–30 mm | Staircase Landing & Feature Wall — all flights and landings |
| GL-01 | Clear glass | Balustrade | Clear frameless laminated toughened glass in recessed floor channel, no handrail rods. Supplier/Product Code to be selected. | 12+12 mm laminated · 1100 mm high | Staircase Landing & Feature Wall — open side of flights and landing |
| MT-01 | Brushed brass | Handrail | Brass-effect brushed PVD / powder-coat champagne bronze slim wall-mounted handrail. Supplier/Product Code to be selected. | Ø40 mm | Staircase Landing & Feature Wall — wall side |
| LT-01 | 2700K LED | Stair lighting | Recessed 2700K LED step lights / under-tread LED strip. Supplier/Product Code to be selected. | — | Staircase Landing & Feature Wall — every 2nd step |
| WF-01 | Warm white paint | Walls | Gypsum skim plaster, smooth level-5 finish | — | Staircase Landing & Feature Wall — stairwell walls |
| WF-01 | Warm white paint | Wall paint | Wall paint: warm white / greige washable matt emulsion (Jotun Fenomastic or equal), 2 coats on primer. Supplier/Product Code to be selected. | — | Staircase Landing & Feature Wall — stairwell walls |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect). Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Staircase Landing & Feature Wall — double-height stair wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Gypsum board ceiling, warm white matt paint, perimeter shadow-gap with 2700K LED cove, trimless downlights; feature pendant to double-height void. Supplier/Product Code to be selected. | — | Staircase Landing & Feature Wall — soffit and void |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Skirting | Stringer skirting in matching marble-look porcelain treads with bullnose nosing | 100 mm | Staircase Landing & Feature Wall — wall stringer |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Shared Bathroom & WCs

Upper floor  ·  Per approved upper floor plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom floor | Marble-look porcelain, matt anti-slip (R10). Supplier/Product Code to be selected. | 600×600 mm · laid to falls | Shared Bathroom & WCs — full floor |
| ST-01 / TL-01 | Beige marble Marble-look porcelain | Bathroom walls | Marble-look porcelain, satin. Supplier/Product Code to be selected. | 1200×600 mm · 9 mm | Shared Bathroom & WCs — full-height wet walls |
| WF-01 | Warm white paint | Wall paint | Moisture- and mould-resistant matt emulsion, warm white, 2 coats on primer. Supplier/Product Code to be selected. | — | Shared Bathroom & WCs — non-tiled wall areas |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Feature wall | Large-format travertine-look porcelain slab cladding (stone-effect) behind vanity with backlit mirror. Supplier/Product Code to be selected. | 1200×2400 mm slabs · 6–12 mm | Shared Bathroom & WCs — vanity wall |
| WF-01 / LT-01 | Warm white paint 2700K LED | Ceiling | Moisture-resistant (MR) gypsum ceiling, anti-mould warm white matt paint, IP44 trimless downlights. Supplier/Product Code to be selected. | 12.5 mm MR board | Shared Bathroom & WCs — full ceiling |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Vanity | Wall-hung vanity, engineered natural oak veneer / oak-effect hpl on mr-mdf, matt, soft-close drawers, LED under-glow. Supplier/Product Code to be selected. | 550 mm deep | Shared Bathroom & WCs — vanity wall |
| ST-01 / TL-01 / GL-01 / LT-01 | Beige marble Marble-look porcelain | Shower area | Marble-look porcelain walls, recessed niche, 8 mm clear tempered frameless glass screen; linear drain with tile insert; concealed thermostatic mixer & ceiling rain head. Supplier/Product Code to be selected. | Screen 10 mm · drain 800 mm | Shared Bathroom & WCs — walk-in shower |
| ST-05 | Quartz | Countertop | Quartz top with undermount ceramic basin, polished. Supplier/Product Code to be selected. | 20 mm | Shared Bathroom & WCs — vanity |
| MT-01 / LT-01 | Brushed brass 2700K LED | Sanitaryware | Wall-hung rimless WC with concealed cistern, white gloss; brass-effect brushed pvd / powder-coat champagne bronze flush plate & mixers. Supplier/Product Code to be selected. | — | Shared Bathroom & WCs — WC and basin |
| MT-01 | Brushed brass | Metal finishes | Brass-effect brushed PVD / powder-coat champagne bronze — taps, shower set, towel rail, accessories. Supplier/Product Code to be selected. | — | Shared Bathroom & WCs — all fittings |
| JN-02 / LT-01 | Natural oak / wood 2700K LED | Doors | Full-height flush door, oak-effect laminate / veneer, concealed frame, concealed hinges, moisture-resistant core. Supplier/Product Code to be selected. | 800×2700 mm | Shared Bathroom & WCs — entry door |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Front Façade & Main Entrance

Exterior & outdoor  ·  Existing massing and proportions unchanged

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-04 / TL-01 / WF-01 | Limestone Marble-look porcelain | Walls / paint | Warm greige textured render across most of the façade, with 60×120 cm stone-look porcelain applied only to the entrance frame and the two ground-floor piers — roughly 20% of the area instead of full limestone. | 120 cm · 20% | Front Façade & Main Entrance — Cladding strategy |
| JN-01 / JN-02 | Smoked walnut Natural oak / wood | Special finish / equipment | Full-height door in an oak-effect aluminium composite panel rather than solid walnut; same proportions and pivot look. | TBC by sample/shop drawing | Front Façade & Main Entrance — Entrance |
| SP-01 | Dark bronze / special | Metal / fittings | Dark powder-coated aluminium window and screen frames, standard profiles. | TBC by sample/shop drawing | Front Façade & Main Entrance — Metalwork |
| LT-01 | 2700K LED | Special finish / equipment | Recessed uplights at the piers, one linear grazer over the entrance, 2700 K, on a single dusk sensor. | 2700 K | Front Façade & Main Entrance — Lighting |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Front Façade & Main Entrance — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Front Façade & Main Entrance — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Front Façade & Main Entrance — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Front Façade & Main Entrance — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Front Façade & Main Entrance — per lighting layout |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Rear Façade & Terrace

Exterior & outdoor  ·  As drawn, opening onto the pool terrace

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| ST-03 / TL-01 | Travertine Marble-look porcelain | Floor / paving | Travertine-look outdoor porcelain, 60×120 cm, laid on sand-cement. | 120 cm | Rear Façade & Terrace — Paving |
| JN-02 / WF-01 | Natural oak / wood Warm white paint | Special finish / equipment | Painted soffit with a slim slat-effect aluminium band instead of a full timber ceiling. | TBC by sample/shop drawing | Rear Façade & Terrace — Soffit |
| LT-01 | 2700K LED | Special finish / equipment | Recessed downlights in the soffit and a continuous LED line at the wall junction. | TBC by sample/shop drawing | Rear Façade & Terrace — Lighting |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Rear Façade & Terrace — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Rear Façade & Terrace — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Rear Façade & Terrace — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Rear Façade & Terrace — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Rear Façade & Terrace — per lighting layout |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Swimming Pool

Exterior & outdoor  ·  11.00 × 5.92 m — exactly the area allocated on the site plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| WF-01 / GL-01 | Warm white paint Clear glass | Walls / paint | Pale sand-tone glass mosaic or premium pool plaster instead of full stone lining, giving the same water colour. | TBC by sample/shop drawing | Swimming Pool — Finish |
| TL-01 | Marble-look porcelain | Floor / paving | 20 mm anti-slip porcelain coping with a flush edge; matching porcelain deck. | 20 mm | Swimming Pool — Coping & deck |
| LT-01 | 2700K LED | Special finish / equipment | Four underwater LEDs plus recessed step lights, 2700 K on the deck. | 2700 K | Swimming Pool — Lighting |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Swimming Pool — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Swimming Pool — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Swimming Pool — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Swimming Pool — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Swimming Pool — per lighting layout |
| ST-04 / TL-01 / GL-01 | Limestone Marble-look porcelain | Pool finish | Pool interior in sand-beige glass mosaic / porcelain, coping in honed limestone-look exterior porcelain, r11 anti-slip. Supplier/Product Code to be selected. | 11.00×5.92 m pool · coping 600×400×30 mm | Swimming Pool — pool shell and coping |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Pool deck | Limestone-look exterior porcelain, R11 anti-slip, anti-slip. Supplier/Product Code to be selected. | 600×600 mm · 20 mm | Swimming Pool — sun deck |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# BBQ & Outdoor Kitchen

Exterior & outdoor  ·  In the position allocated on the site plan, under an architectural canopy

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| TL-01 | Marble-look porcelain | Countertops / stone | Blockwork carcass clad in outdoor porcelain with a 20 mm porcelain worktop; built-in gas BBQ, stainless sink and an under-counter fridge. | 20 mm | BBQ & Outdoor Kitchen — Counter |
| JN-02 / WF-01 / MT-01 | Natural oak / wood Warm white paint | Special finish / equipment | Powder-coated steel frame with a painted soffit and an aluminium slat band, replacing the bronze-framed timber canopy. | TBC by sample/shop drawing | BBQ & Outdoor Kitchen — Canopy |
| LT-01 | 2700K LED | Special finish / equipment | Task strip under the canopy edge plus one warm downlight per bay. | TBC by sample/shop drawing | BBQ & Outdoor Kitchen — Lighting |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | BBQ & Outdoor Kitchen — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | BBQ & Outdoor Kitchen — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | BBQ & Outdoor Kitchen — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | BBQ & Outdoor Kitchen — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | BBQ & Outdoor Kitchen — per lighting layout |
| ST-05 | Quartz | Outdoor countertop | Flamed / leathered dark granite or exterior sintered stone, UV stable. Supplier/Product Code to be selected. | 30 mm | BBQ & Outdoor Kitchen — BBQ counter with sink & fridge |
| JN-02 / MT-01 | Natural oak / wood Brushed brass | Outdoor cabinets | Marine-grade stainless steel carcasses with dark bronze / wood-effect HPL doors. Supplier/Product Code to be selected. | — | BBQ & Outdoor Kitchen — base units |
| JN-02 / MT-01 | Natural oak / wood Brushed brass | Canopy | Architectural canopy, dark bronze aluminium with wood-effect slat soffit. Supplier/Product Code to be selected. | Per drawing | BBQ & Outdoor Kitchen — over BBQ |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Outdoor Lounge

Exterior & outdoor  ·  Raised platform at the north-east of the site plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| SP-01 | Dark bronze / special | Special finish / equipment | Modular aluminium-and-rope lounge with quick-dry cushions in sand; ceramic-top low tables. | TBC by sample/shop drawing | Outdoor Lounge — Furniture |
| JN-02 | Natural oak / wood | Special finish / equipment | Powder-coated aluminium pergola with fixed slats instead of a hardwood structure. | TBC by sample/shop drawing | Outdoor Lounge — Structure |
| TL-01 / JN-02 | Marble-look porcelain Natural oak / wood | Floor / paving | Wood-effect outdoor porcelain rather than hardwood decking — no annual oiling. | TBC by sample/shop drawing | Outdoor Lounge — Deck |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Outdoor Lounge — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Outdoor Lounge — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Outdoor Lounge — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Outdoor Lounge — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Outdoor Lounge — per lighting layout |
| JN-02 / MT-01 / TX-01 | Natural oak / wood Brushed brass | Pergola | Dark bronze aluminium pergola with wood-effect louvres, outdoor-fabric upholstery in sand / taupe. Supplier/Product Code to be selected. | Per drawing | Outdoor Lounge — seating area |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Outdoor Dining

Exterior & outdoor  ·  Adjacent to the BBQ zone as drawn

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| SP-01 | Dark bronze / special | Countertops / stone | 2.40 m aluminium table with a ceramic top and eight woven dining chairs. | 2.40 m | Outdoor Dining — Table & seating |
| SP-01 | Dark bronze / special | Special finish / equipment | Extension of the BBQ canopy rather than a separate structure — one build, two uses. | TBC by sample/shop drawing | Outdoor Dining — Shade |
| LT-01 | 2700K LED | Special finish / equipment | Two warm pendants under the canopy on a dimmer. | TBC by sample/shop drawing | Outdoor Dining — Lighting |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Outdoor Dining — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Outdoor Dining — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Outdoor Dining — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Outdoor Dining — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Outdoor Dining — per lighting layout |
| JN-02 / MT-01 / TX-01 | Natural oak / wood Brushed brass | Pergola | Dark bronze aluminium pergola with wood-effect louvres, outdoor-fabric upholstery in sand / taupe. Supplier/Product Code to be selected. | Per drawing | Outdoor Dining — seating area |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Children's Play Area

Exterior & outdoor  ·  Dedicated zone in the position shown on the site plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| FL-01 | EPDM rubber | Special finish / equipment | Bonded rubber safety surface in sand tone — the same product family as Option 1, in a plain colour. | TBC by sample/shop drawing | Children's Play Area — Surfacing |
| JN-02 | Natural oak / wood | Special finish / equipment | Catalogue climbing frame and swing in timber-effect finish rather than a bespoke oak structure. | TBC by sample/shop drawing | Children's Play Area — Equipment |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Low hedge screen from the pool deck and two bollard lights. | TBC by sample/shop drawing | Children's Play Area — Screening & lighting |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Children's Play Area — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Children's Play Area — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Children's Play Area — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Children's Play Area — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Children's Play Area — per lighting layout |
| FL-01 | EPDM rubber | Safety flooring | Wet-pour EPDM rubber safety surface, sand / sage colours. Supplier/Product Code to be selected. | 40–80 mm (per fall height, EN 1177) | Children's Play Area — whole play zone |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.

OPTION 2 · AFFORDABLE MODERN LUXURY

# Garden, Walkways & Lighting

Exterior & outdoor  ·  Full plot as shown on the site plan

| REF. | SAMPLE | ITEM | MATERIAL · COLOUR · FINISH | SIZE / THICKNESS | EXACT LOCATION |
| --- | --- | --- | --- | --- | --- |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Six repeated species: olive, frangipani, clipped ficus hedge, agave, lantana groundcover and buffalo lawn. Fewer semi-mature specimens, more small stock allowed to grow in. | TBC by sample/shop drawing | Garden, Walkways & Lighting — Planting |
| TL-01 | Marble-look porcelain | Special finish / equipment | Outdoor porcelain pavers on compacted base with gravel margins instead of continuous natural stone. | TBC by sample/shop drawing | Garden, Walkways & Lighting — Walkways |
| WF-01 / GL-01 / LS-01 | Warm white paint Clear glass | Special finish / equipment | Built-in rendered planters painted to match the façade, plus fibreglass pots at the entrance. | TBC by sample/shop drawing | Garden, Walkways & Lighting — Planters |
| LT-01 / LS-01 | 2700K LED Planting | Special finish / equipment | Eight tree uplights, bollards along the pedestrian route and step lights, all 2700 K on one dusk-to-midnight circuit. | 2700 K | Garden, Walkways & Lighting — Lighting |
| TL-01 / WF-01 | Marble-look porcelain Warm white paint | Special finish / equipment | Rendered boundary wall with a porcelain-clad gate frame; dark powder-coated aluminium sliding vehicle gate and matching pedestrian gate. | TBC by sample/shop drawing | Garden, Walkways & Lighting — Boundary & gates |
| ST-04 / TL-01 | Limestone Marble-look porcelain | Outdoor paving | Limestone-look exterior porcelain, R11 anti-slip. Supplier/Product Code to be selected. | 600×600 / 900×600 mm · 20–30 mm | Garden, Walkways & Lighting — paths, terraces and hard landscape |
| ST-04 / WF-01 | Limestone Warm white paint | Outdoor walls | Natural beige limestone cladding with exterior acrylic render, warm sand, matt. Supplier/Product Code to be selected. | 30 mm stone · render 15 mm | Garden, Walkways & Lighting — façades, boundary and feature walls |
| MT-01 | Brushed brass | Outdoor metal | Dark bronze powder-coated aluminium (gates, pergola, frames, trims). Supplier/Product Code to be selected. | — | Garden, Walkways & Lighting — all exterior metalwork |
| JN-02 | Natural oak / wood | Outdoor timber | Composite wood-effect decking, teak tone. Supplier/Product Code to be selected. | 140×21 mm boards | Garden, Walkways & Lighting — decks, cladding and slatted screens |
| MT-01 / LT-01 | Brushed brass 2700K LED | Outdoor lighting | 2700K IP65 bollards, step lights, uplights and linear wall-washers, dark bronze. Supplier/Product Code to be selected. | — | Garden, Walkways & Lighting — per lighting layout |
| ST-04 / WF-01 / LS-01 | Limestone Warm white paint | Softscape | Natural lawn, olive / palm feature trees, low hedges, raised planters in render / limestone, drip irrigation. Supplier/Product Code to be selected. | — | Garden, Walkways & Lighting — planting beds and lawn |

Status: Approved design unless marked TBC. Reference codes are project schedule identifiers only.